3GPP TSG-SA3 Meeting #109 	S3-230706

# Athens,Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1532 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on authorization for inter NF mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Existing 3GPP procedures lack the means to check whether subscriptions created at a source NF (e.g. source AMF) are authorised at a target NF (e.g., target AMF) upon inter-NF (e.g. inter-AMF) context transfer (e.g. inter-AMF mobility of a UE). This can result in the target NF sending notifications to the NFc that are not permitted by the operator. This is exemplified below. NFc (e.g., AF via NEF or any other NF) creates a subscription at AMF-A. NFc obtains an authorization token to consume the AMF-A service and provides the token to AMF-A certifying that NFc is authorised to subscribe to the requested events to AMF-A and that, accordingly, AMF-A is authorised to send event notification (data) to NFc. An example use case may be an advertising company that wants to push some data to UE by asking AMF to notify UE location change. If UE moves from AMF-A/PLMN-A to AMF-B/PLMN-B, all NFc(s) subscriptions are also transferred from AMF-A to AMF-B as a part of UE context transfer. As per existing 3GPP specifications, AMF-B proceeds with sending notifications to NFc. However, it can be so that the operator (of AMF-B) does not authorise NFc to create a subscription/receive notification from AMF-B. Thus, there is a gap in AMF-to-AMF mobility with regard to authorization, which applies equally to intra PLMN handovers where the access token is per target NFp Instance id or target NFp Set. This problem also applicable to the NWDAF-to-NWDAF mobility where subscription created at the NWDAF1 are transferred to NWADAF2 as a part of context transfer. Please refer to the 23.288, section 6.1b Please refer to the discussion paper S3-230704 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new section 13.4.1.x that explains Oauth enhancement for NF-to-NF mobility |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | During AMF to AMF mobility or NWDAF to NWDAF mobility, subscriptions are transferred to the target AMF/NWDAF and target AMF/NWDAF starts sending the notification to the without an authorization. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF CHANGES

#### 13.4.1.x	Service access authorization in inter NF mobility scenario

During inter-NF mobility, i.e., AMF to AMF mobility and/or NWDAF to NWDAF mobility, all subscriptions created/stored at the source NF are transferred to the target NF. The source NF shall store the access token along with the created subscriptions and forward the same to the target NF as a part of context transfer and by letting the target NF do the authorization checking:

- if a token is received for the subscription from the source NF and authorizes the subscription to be served by the target NF, the target NF shall proceed with accepting the subscription and generating data (notification) to the NFc.

- if no token is received for the subscription from the source NF, or if a token is received but does not authorize the subscription to be served by the target NF, the target NF shall trigger the NFc to re-authorize the subscription at the target NF.

************ END OF CHANGES