3GPP TSG-SA3 Meeting #110 	S3-230721

# Athens, Greece, 20 - 24 February 2023

# Source:	MITRE

# Title:	New KI: Protection of Home Network Identifiers

# Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

This pCR proposes a new KI for TR 33.870: Protection of Home Network Identifiers

# 2	References

[1]	3GPP TR 33.870 v0.5.0 "Study of privacy of identifiers over radio access"

# 3	Rationale

The SUCI aims to protect the privacy of user’s permanent identifier SUPI. But the Home Network Identifier and Routing Indicator part of the SUPI are not concealed in the SUCI. This may lead to other privacy issues, whereby an adversary listening on the air interface can identify and track subscribers with a sensitive Home Network Identifier. For example, a public safety official (e.g., police, firefighter) with a UE subscribed to a public safety PLMN may be distinguishable from nearby UEs or subscribers for a 5G private network.

# 4	Detailed proposal

Approve the following proposed changes

********************** Start of 1st Change ****************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	3GPP TS 24:501: “Non-Access-Stratum (NAS) protocol for 5G System (5GS)”.

[3]	3GPP TR 33.501: “Security architecture and procedures for 5G system”.

[4]	IETF RFC 3629: “UTF-8, a transformation format of ISO 10646”.

[5]	IETF RFC 7542: “The Network Access Identifier”.

[6]	3GPP TS 33.220: “Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)”.

[X]	3GPP TS 23.003: “Numbering, addressing and identification”.

********************** End of 1st Change ****************************

********************** Start of 2nd Change ****************************

## 5.X	Key Issue X: Protection of Home Network Identifiers

### 5.X.1	Key issue details

Subscription Concealed Identifier (SUCI) is a privacy preserving identifier containing the concealed Subscription Permanent Identifier (SUPI). SUCI is composed of 6 parts (i.e., SUPI Type, Home Network Identifier, Routing Indicator, Protection Scheme Identifier, Home Network Public Key Identifier, Scheme Output) as specified in clause 6.12.2 of TS 33.501 [3]. The Home Network Identifier and Routing Indicator are defined in clause 2.2B of TS 23.003 [X] and are used to identify and route communication to the home network of the subscriber.

However, the Home Network Identifier and Routing Indicator part of the SUPI are not concealed in the SUCI. This may lead to other privacy issues, whereby an adversary listening on the air interface can identify and track subscribers with a sensitive Home Network Identifier. For example, a public safety official (e.g., police, firefighter) with a UE subscribed to a public safety PLMN may be distinguishable from nearby UEs or subscribers for a 5G private network.

### 5.X.2	Security threats

An adversary listening on the air interface can identify and track subscribers with sensitive Home Network Identifiers.

Since Home Network Identifiers may be public information, the adversary can perform a targeted attack on subscribers of a particular Home Network (e.g., public safety PLMNs) or subscribers of a private network.

### 5.X.3	Potential security requirements

Solution should provide an option for protection of Home Network Identifiers in the SUCI on the air interface.

********************** End of 2nd Change ****************************