3GPP TSG-SA WG3 Meeting #110	S3-230731

Athens, GR, 20 – 24 February 2023                                                   revision of S3-20xabc

Source:	InterDigital

Title:	PCR to 33.870 New clause for comparative evaluation of KI#1 solutions

Document for:	Approval

Agenda Item:	5.4

Work Item / Release:	FS_Id_Prvc

This contribution proposes the addition of a new clause for comparative evaluation of KI#1 solutions omparative evaluation of KI#1 solutions.

**** END OF CHANGE 1 ****

# 1	Decision/action requested

This pCR is proposing the addition of a new clause containing comparative evaluation of solutions addressing KI#1 to TR 33.870 SA3 is kindly requested to approve this document.

# 2	References

[1]	3GPP TR 33.870 v0.4.0

# 3	Rationale

This contribution proposes to add a new clause, Comparative evaluation of solutions addressing KI#1, to TR 33.870.

# 4	Detailed proposal

**** START OF CHANGE 1 ****

# 6	Solutions

## 6.X	Comparative evaluation of solutions addressing KI#1

Table 6.X-1: Comparative evaluation of solutions addressing KI#1

| Sol. # | Type | Addr. KI#1? | Evaluation Condition 1 | Evaluation Condition 2 | Impact |  | Re-provisioning |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | UE | Network | UE | Network |  |
| 1 | configuration | Y | Yes Supports 5G-AKA and EAP-AKA' mechanisms using profile A, profile B, or proprietary SUCI calculation scheme as well as other key generating EAP methods, e.g., EAP-TLS and EAP-TTLS. | No Does not support backward compatibility with SUPIs in NAI format, which might already be deployed. | No | No | Yes | Yes |  |
| 2 | padding | Y | Yes Supports 5G-AKA and EAP-AKA' mechanisms using profile A, profile B, or proprietary SUCI calculation scheme as well as other key generating EAP methods, e.g., EAP-TLS and EAP-TTLS. | Yes Supports backward compatibility with SUPIs in NAI format, which might already be deployed | Yes | Yes | No | No | Uses random length padding. |
| 3 | pseudonym |  | Yes, partially Supports 5G-AKA and EAP-AKA' mechanisms using profile A, profile B, or proprietary SUCI calculation scheme Does not support other key generating EAP methods, e.g., EAP-TLS and EAP-TTLS. | Yes Utilizes preconfigured SUPI pseudonyms instead of SUPI. Does not require changes for SUPIs in NAI format, which might already be deployed | Yes | Yes | Yes | Yes | Uses preconfigured SUPI pseudonyms |
| 4 | configuration | *Yes | Yes Supports 5G-AKA and EAP-AKA' mechanisms using profile A, profile B, or proprietary SUCI calculation scheme as well as other key generating EAP methods, e.g., EAP-TLS and EAP-TTLS. | No Does not support backward compatibility with SUPIs in NAI format, which might already be deployed. | No | No | Yes | Yes | *Assumes that SUPIs can be truncated to a uniformal lengths and addresses the requirement only partially. Unclear if usernames can be truncated and still preserve their uniqueness. |
| 5 | configuration | Y | Yes Supports 5G-AKA and EAP-AKA' mechanisms using profile A, profile B, or proprietary SUCI calculation scheme as well as other key generating EAP methods, e.g., EAP-TLS and EAP-TTLS. | No Does not support backward compatibility with SUPIs in NAI format, which might already be deployed. | No | Yes | Yes | No |  |
| 6 | configuration | Y | Yes Supports 5G-AKA and EAP-AKA' mechanisms using profile A, profile B, or proprietary SUCI calculation scheme as well as other key generating EAP methods, e.g., EAP-TLS and EAP-TTLS. | No Does not support backward compatibility with SUPIs in NAI format, which might already be deployed. | No | No | Yes | Yes | Uses random length padding for pre-configured SUPIs. |
| 7 | truncation | *Y | Yes Supports 5G-AKA and EAP-AKA' mechanisms using profile A, profile B, or proprietary SUCI calculation scheme as well as other key generating EAP methods, e.g., EAP-TLS and EAP-TTLS. | Yes Supports backward compatibility with SUPIs in NAI format, which might already be deployed | Yes | Yes | No | No | Uses random length truncation. *Unclear if truncation can preserve the uniqueness of username. |
| 8 | configuration | Y | Yes Supports 5G-AKA and EAP-AKA' mechanisms using profile A, profile B, or proprietary SUCI calculation scheme as well as other key generating EAP methods, e.g., EAP-TLS and EAP-TTLS. | No Does not support backward compatibility with SUPIs in NAI format, which might already be deployed. | No | No | Yes | Yes | provisioning |
| 9 | padding | Y | Yes Supports 5G-AKA and EAP-AKA' mechanisms using profile A, profile B, or proprietary SUCI calculation scheme as well as other key generating EAP methods, e.g., EAP-TLS and EAP-TTLS. | Yes Supports backward compatibility with SUPIs in NAI format, which might already be deployed | Yes | Yes | No | No | Uses static padding. |
| 10 | pseudonym | Y | Yes Supports 5G-AKA and EAP-AKA' mechanisms using profile A, profile B, or proprietary SUCI calculation scheme as well as other key generating EAP methods, e.g., EAP-TLS and EAP-TTLS. | Yes Supports backward compatibility with SUPIs in NAI format, which might already be deployed | Yes | Yes | No | Yes | Uses SUPI hashing on the UE to create a static-length SUPI pseudonym.. |

Solutions 1, 4, 5, 6, and 8 use configuration.

Solutions 2 and 9 use dynamic padding

Solutions 3 and 10 use pseudonyms

Solution 7 uses dynamic truncation.

**** END OF CHANGE 1 ****