3GPP TSG-SA WG3 Meeting #110	S3-230742

Athens, GR, 20 – 24 February 2023                                                   revision of S3-20xabc c

Source:	InterDigital

Title:	PCR to 33.870 - Solution #3 Evaluation

Document for:	Approval

Agenda Item:	5.4

Work Item / Release:	FS_Id_Prvc

This contribution proposes an evaluation for Solution #3 in TR 33.870.

**** END OF CHANGE 1 ****

# 1	Decision/action requested

This pCR is proposing content for the Evaluation section of Solution #3.

SA3 is kindly requested to approve this document.

# 2	References

[1]	3GPP TR 33.870 v0.5.0

# 3	Rationale

This contribution proposes changes to Solution #3 in TR 33.870. It adds content to the Evaluation section.

Clause 5.1.3 of this document defines the following conditions for proper evaluation of a solution addressing KI#1

-	the solution needs to indicate which authentication mechanisms it works with and whether that authentication mechanism preserves SUPI length.

-	the solution needs to be evaluated as to whether it is backwards compatible with SUPIs in NAI format, which might already be deployed.

The proposed evaluation is based on the evaluation conditions defined in clause 5.1.3.

# 4	Detailed proposal

**** START OF CHANGE 1 ****

### 6.3.3	Evaluation

The following evaluation is based on the evaluation conditions defined in clause 5.1.3.

The solution addresses the requirement of key issue 1, Privacy aspects of variable length user identifiers.

This solution is providing a means to privacy-protect, i.e., make encrypted lengths of identical SUPIs in NAI format unrecognizable to the attacker while supporting 5G-AKA and EAP-AKA' mechanisms using profile A, profile B, or proprietary SUCI calculation scheme but does not support other key generating EAP methods, e.g., EAP-TLS and EAP-TTLS.

While solution uses pre-provisioned pseudonyms as overlayed subscription identifier, it does not require changes in the operator’s SUPI allocation plan and does support backward compatibility with SUPIs in NAI format, which might already be deployed.

The solution relies on the use of a preconfigured pseudonym instead of the SUPI during SUCI calculation. The pseudonyms are under the control of the operator who, via choosing appropriate length for the pseudonyms,    steer the overall length distribution of the identifier sent over the air and achieve the desired anonymity goal.

The solution has impact on the UE and the SIDF, consequently the UDM. The solution requires the introduction of new protection scheme identifiers to signal the use of pseudonyms.

The solution does not address the privacy issue related to the usage of the identity request in the EAP protocol. Therefore, the solution only addresses the issue as long as the identifier retrieval by the EAP method is not required. For example, this is not the case when anonymous SUCI is used.

Editor's Note: Further evaluation related to the use of new scheme identifier are FFS.

The solution has the following UE and Core Network impacts:

UE: The UE needs to be provisioned with a fixed length digital identifier by reusing existing mechanism that supports provisioning of Routing ID and other information (e.g., related to SUCI construction information). Further the SUPI type should be set as digital identifier based NAI type.

UDM: The UDM needs to store the actual NAI based SUPI along with the associated fixed length digital identifier. On SUCI deconcealment, the UDM needs to fetch the related NAI based SUPI using the pseudonym. To do so, the UDM/UDR internal structure has to be amended (e.g., new tables or columns to allow subscription records to be looked up based on the pseudonym).

Editor’s Note: Further impact on UE and evaluation is FFS.

Editor’s Note: The usage of fixed length identifier and its impact to the certificate related to TLS authentication method is FFS.

**** END OF CHANGE 1 ****