3GPP TSG-SA3 Meeting #110	S3-230810

# Athens, Greece, 20 - 24 February 2023

# Source:	Qualcomm Incorporated

# Title:	Updates on the solution #23

# Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes to update the solution #23 in TR 33.740.

# 2	References

[1]	TS 33.503

[2]	TR 33.740 v0.5.0

# 3	Rationale

This contribution proposes to update the solution #23.

In particular, this contribution proposes to provision direct discovery set protection indication to source UE and target UE when an RSC and the corresponding security materials are provisioned at those UEs. The direct discovery set protection indication informs the UE of whether to enable/disable the direct discovery set(s) protection, thus, enables a source UE/target UE to determine whether to do/undo the protection of direct discovery set(s) during the U2U relay discovery procedure. The direct discovery set protection indication is set to be enabled when the U2U relay service identified by the RSC supports to relay multiple direct discovery set(s) associated with different ProSe services. Otherwise, it is set to be disabled.

In addition, since the solution clarified when two sets of discovery security materials are needed and used, the below two Editor’s Notes are no longer needed. Thus, it is proposed to delete the below two Editor’s Notes.

Editor’s Note: The need of two sets of security parameters is FFS.

Editor’s Note: It's FFS whether ProSe services requiring different security materials share the same RSC.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [2].

***** START OF CHANGES *****

## 6.23	Solution #23: Security mechanism for UE-to-UE Relay Model A discovery

### 6.23.1	Introduction

This solution addresses the Key Issue #1.

An Announcement message by the U2U relay includes two sets of elements, i.e., direct discovery set(s) (e.g., the User Info ID of source UE and target UE) and a U2U discovery set (e.g., Type of Discovery Message, RSC, User Info ID of the relay). The 5G ProSe U2U Relay only modifies the elements of the U2U discovery set. The direct discovery set is constructed by the target UE and is only interpreted by the source UE. The U2U Relay can use a single RSC (and the associated security materials) to relay direct discovery sets associated with the multiple ProSe services. This means that the 5G ProSe U2U Relay uses the same security materials to protect the Announcement messages that contain the direct discovery sets associated with different ProSe services. Unless each direct discovery set is protected using the security materials of the corresponding ProSe service, a source UE (or a target UE) that is authorized to use the RSC for U2U discovery can decrypt (and manipulate) any direct discovery sets that are delivered using the same RSC. This poses security threats.

To prevent such security threats, this solution supports both scenarios by provisioning direct discovery set protection indication to the source UE and target UE. In case a single RSC is used to relay multiple direct discovery sets associated with different ProSe services, the Announcement message is protected using two sets of security materials (i.e., the Announcement message and the direct discovery set(s) are protected using the respective set of security materials).

In case a single RSC is used to relay direct discovery sets associated with a single ProSe service, the Announcement message is protected using a single set of security materials associated with the RSC.

### 6.23.2	Solution details

Figure 6.23.2-1: Model A discovery

0. This solution consists of two protection mechanisms using two sets of security materials: one for the direct discovery set(s) protection and the other one for the Announcement message protection. In addition, the source UE and target UE are provisioned with the direct discovery set protection indication informing whether to enable or disable the direct discovery set(s) protection.

1. Direct discovery set(s) protection by the target UE:

The source UE and target UEs are provisioned with the ProSe restricted code and the discovery security materials associated with the ProSe restricted code as specified in clause 6.1.3.2.2 of TS 33.503 [6].

The target UE protects the direct discovery set(s) using the discovery security materials associated with the ProSe restricted code if the direct discovery set protection indication is set to be enabled.

The target UE provides the direct discovery set(s) to the U2U Relay.

Editor’s Note: how the target UE provides the protected direct discovery set(s) to the U2U Relay needs to be aligned with the U2U relay Model A discovery procedure defined by SA2.

Editor’s Note: The need of two different discovery codes (ProSe restricted code and RSC) is FFS.

NOTE: the protection mechanisms specified in clause 6.1.3.2.3 of TS 33.503 [6] are reused for the direct discovery set and U2U discovery set protection. The details of how the protection mechanisms in clause 6.1.3.2.3 of TS 33.503 [6] is applied to the U2U discovery message protection will be specified during the normative work.

2. Announcement message protection by U2U Relay:

The U2U Relay and source UE are provisioned with the RSC for a U2U relay service and the discovery security materials associated with the RSC as specified in clause 6.1.3.2.2 of TS 33.503 [6].

The U2U Relay protects an Announcement message using the provisioned discovery security materials associated with the RSC as specified in clause 6.1.3.2.3 of TS 33.503 [6]. The Announcement message contains the direct discovery set(s) provided by the target UEs.

The source UE decrypts and/or verifies the received Announcement message using the discovery security materials associated with the RSC. The source UE extracts the direct discovery set(s) from the Announcement message. If the direct discovery set protection indication is set to be enabled, the source UE decrypts and/or verifies the direct discovery set(s) using the discovery security materials associated with the ProSe restricted code.

### 6.23.3	Evaluation

This solution addresses the Key Issue #1. In particular, this solution supports both scenarios by provisioning direct discovery set protection indication to the source UE and target UE:

- Two sets of discovery security materials are used to protect direct discovery set and the discovery messages in case a single RSC is used by a 5G ProSe U2U Relay to relay multiple direct discovery sets associated with different ProSe services.

- A single set of discovery security materials is used to protect the discovery messages in case a single RSC is used by a 5G ProSe U2U Relay to relay direct discovery set(s) associated with a single ProSe service.

Editor’s Note: Further evaluation is FFS.

***** END OF CHANGES *****