3GPP TSG-SA3 Meeting #110	S3-230812

# Athens, Greece, 20 - 24 February 2023

Title:	draft LS on clarification on ProSe UE-to-UE relay discovery direct discovery set

Response to:

Release:	Release 18

Work Item:	FS_5G_ProSe_Ph2

Source:	Qualcomm Incorporated (To be SA3 after approval)

To:	SA2

Cc:

Contact person:	Hongil Kim

Qualcomm Incorporated

hongilk@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

According to the conclusion of Key Issue #1 (Support of UE-to-UE Relay) in TR 23.700-33, 5G ProSe UE-to-UE relay discovery message contains two sets of elements, i.e., direct discovery set(s) and a U2U relay discovery set. In addition, the direct discovery set of elements can be part of the contents of 5G ProSe Direct Discovery message as defined in Rel-17.

Based on the above conclusion, SA3 has been discussing potential security mechanisms for 5G ProSe UE-to-UE Relay discovery message protection. Currently, there are two approaches:

- Protection of direct discovery set and U2U relay discovery messages separately using two different sets of discovery security materials, where one set is used to protect the direct discovery set that is associated with a specific ProSe service and the other set is used to protect the U2U relay discovery message that is associated with the RSC

- Protection of the U2U relay discovery messages using a single set of discovery security materials associated with the RSC.

Which approach to use depends on whether multiple direct discovery sets that correspond to different ProSe services (i.e., ProSe identifiers) can be carried over a single U2U relay service (i.e., a single RSC). However, the conclusion in SA2 is not clear on this aspect. Therefore, SA3 asks SA2 to provide clarification on whether multiple direct discovery sets that correspond to different ProSe services can be carried over a single U2U relay service.

# 2	Actions

To SA2, CT1

ACTION: 	SA3 kindly asks SA2 to provide clarification for the above use case regarding U2U relay.

# 3	Dates of next TSG SA WG 3 meetings

SA3#110Bis-e	17 - 21 April 2023	Electronic meeting

SA3#111	22 - 26 May 2023	EU (TBD)