3GPP TSG-SA3 Meeting #110	S3-230817

# Athens, Greece, 20 - 24 February 2023

# Source:	Qualcomm Incorporated

# Title:	Updates to the Key Issue #5

# Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

This contribution proposes to update the Key Issue #5 in TR 33.893.

# 2	References

[1]	TR 33.893 v0.5.0

# 3	Rationale

This contribution proposes to update the Key Issue #5. Particularly, this contribution proposes to add a corresponding security requirement based on the security threats.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [1].

***** START OF CHANGES *****

## 5.5	Key issue #5: Protection of groupcast/broadcast

### 5.5.1	Key issue details

Editor’s Note: This key issue details may need to be updated to align with the conclusion in RAN2.

In TR 38.859 [10], it is mentioned that using SLPP unicast messages between UEs is the baseline for SL positioning. In addition, sending part of SLPP positioning signalling among UEs via broadcast/groupcast is also possible:

“Unicast/one-to-one operation is assumed as baseline for exchange of SLPP signaling between UEs. Unicast SLPP session-based operation is supported. At least “centralized” operation is supported, i.e., operation where one UE performs range and/or position calculations based on measurement/location information relating to itself and/or other UEs. It is feasible to send at least the following positioning signaling for groupcast/broadcast (in addition to unicast) from RAN2’s perspective:

- SL positioning capability

- SL positioning assistance data

Location information is not excluded and can be further considered in normative work.”

Furthermore, in TR 38.859 [10], it is also mentioned that security issues should be considered on how to protect the SL groupcast/broadcast messages:

“RAN2 will further discuss in normative work:

-	The security issues (e.g., requirements for ciphering and/or integrity) on specific information of SL positioning capability and assistance data in groupcast/broadcast.

-	The use cases for applying groupcast/broadcast.”

### 5.5.2	Security threats

Failures to protect SL groupcast/broadcast communications will lead to the following threats:

-	Passive attackers can eavesdrop on privacy sensitive data exchanged between UEs.

-	Active attackers can intercept, modify or replay data packets exchanged between UEs.

-	The broadcasting/groupcasting UE may be impersonated by an attacker.

### 5.5.3	Potential security requirements

The 5G System shall support a means to provide confidentiality, integrity and replay protection of  groupcast/broadcast SLPP messages.

***** END OF CHANGES *****