3GPP TSG-SA3 Meeting #110 	S3-230846

Athens, Greece, 20 - 24 February 2023

Title:	Draft Reply LS on Authorization of NF service consumers for data access via DCCF

Response to:	LS C4-225161 on Authorization of NF service consumers for data access via DCCF from CT4

Release:	Rel-17

Work Item:	eNA_PH2

Source:	SA3#110

To:	CT4

Cc:	CT3

Contact person:	German Peinado

german.peinado@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	(final CR to be attached)

# 1	Overall description

In CT4#113 (Toulouse) meeting CT4 sent an LS (C4-225161) to SA3 pointing out important misalignments between requirements specified in TS 29.510 and Annex X of TS 33.501 on Authorization of NF service consumers (e.g., NWDAF) for data access via DCCF.

SA3 acknowledges the indicated misalignments and agreed the following corrections/clarifications in Annex X of TS 33.501:

- From Rel-15 onwards: the NF Instance ID (nfInstanceId IE) in the access token request and the subject claim in the access token will refer to the intermediate NF service consumer (e.g., DCCF).

- From Rel-17 onwards: the source NF Instance ID (sourceNfInstanceID IE) in the access token request and the sourceNfInstanceId claim in the access token will refer to the source NF service consumer (e.g., NWDAF).

The changes are captured in the CR S3-230844.

The CR requires from CT 4 the definition of a new header to carry the source NF client credential from the DCCF to the NFp for backwards compatibility issue with pre-release 17 NFp.

# 2	Actions

To CT4 group

ACTION:  SA3 kindly asks CT4 to take note of the TS 33.501 Annex X update, and proceed with the definition of the new header required to solve the backwards compatibility issue.

# 3	Dates of next TSG SA WG 3 meetings

SA3#110Bis-e	17 - 21 April 2023	Electronic meeting

SA3#111	22 - 26 May 2023	EU (TBD)