# 3GPP TSG-SA3 Meeting #110 	S3-230890

# Athens, Greece, 20 - 24 February 2023

# Source:	ZTE

# Title:	Security Assurance Requirement and Test for AKMA context removal

# Document for:	Approval

Agenda Item:	4.5

1	Decision/action requested

SA3 is kindly asked to approve the added security assurance requirement and related test case for SCAS AAnF.

2	References

[1]	3GPP TS 33.535 “Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)”

3	Rationale

Based on the requirement in clause 6.6 of TS 33.535, this contribution proposes to add a security assurance requirement and the corresponding test case to TS 33.537 to assure correct handling by the AAnF in case of AKMA subscription data and AKMA context synchronization.

4	Detailed proposal

*************Start of Change **************

4.2.2	Security functional requirements on the AAnF deriving from 3GPP specifications and related test cases

4.2.2.x 	Authentication and key agreement procedure

4.2.2.x.1	AKMA context removal

Requirement Name: AKMA context removal

Requirement Reference: TS 33.535 [4], clause 6.6

Requirement Description: "This procedure is used to remove the AKMA context in the AAnF. NF consumers may initiate this procedure due to local policy."  as specified in TS 33.535[4], clause 6.6.

Threat References: TBD.

Test Case:

Test Name: TC_AKMA context removal_AAnF

Purpose:

Verify that AKMA context removal operation is correctly handled by the AAnF.

Pre-Conditions:

- Test environment contains a UE with its AKMA capabilities, AUSF, AF and OAM. The UE, AUSF, AF, and OAM can be simulated.

- AAnF network product is connected in emulated/real network environment.

- AAnF stores the AKMA context of the UE after primary authentication.

- The AKMA subscription data of the UE is deactivated.

Execution Steps

- The OAM sends a Naanf_AKMA_Context_Remove request to AAnF.

2)	AAnF deletes AKMA Context (e.g. SUPI, A-KID and KAKMA) from its local database base on the SUPI.

3)	AAnF sends a Naanf_AKMA_Context_Remove response to OAM.

Expected Results:

The tester cannot find the AKMA context of the SUPI in AAnF. Evidence suitable for the interface, e.g., Screenshot containing the operational results.

*************End of Change **************