3GPP TSG-SA3 Meeting #110 	S3-230894

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.222 | CR | draftCR | rev | <-> | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General part for GBA DTLS to TS 33.222 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA_GBA_DTLS |  |  |  |  |  | Date: |  |  | 2023-2-6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In order to complete the work on the DTLS for GBA Ua protocol, it is required to add some general requirement for UE and NAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add some general requirement for UE and AF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete work for the DTLS for GBA Ua protocol |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of Change 1****

Annex X (normative): 
DTLS profiles for  GBA Ua

X.1	DTLS based protocols

X.1.1	General

This Annex covers the aspects specific to the DTLS profiles for GBA Ua.

The Constrained Application Protocol (CoAP) is a light-weight protocol defined by IETF in RFC 7252 [xx] and designed specifically for application layer communication for constrained devices.  CoAP is designed to easily interface with HTTP for integration with the Web while meeting specialized requirements such as multicast support, very low overhead, and simplicity for constrained environments. RFC 9202 [xy] specifies CoAP over DTLS.

X.1.1.1	Requirement on the UE

If CoAP is used, the UE hosts the CoAP client and the DTLS client. The CoAP client and DTLS client either resides both in the ME or in the UICC. The CoAP+DTLS client may reside in the ME or in the UICC or both might host an CoAP+DTLS client independently of each other.

X.1.1.2	Requirement on the NAF

CoAP and DTLS should be supported by the NAF for the UE-NAF reference point (Ua).

X.1.2	Shared key-based UE authentication with certificate-based AF authentication

X.1.3	Shared key-based mutual authentication between UE and AF

**** End of Change 1****

**** Start of Change 2****

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.002: "Network architecture".

[2]	3GPP TS 22.250: "IP Multimedia Subsystem (IMS) group management"; Stage 1".

[3]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture".

[4]	Void

[5]	3GPP TS 33.141: "Presence Service; Security".

[6]	Void.

[7]	Void.

[8]	Void.

[9]	IETF RFC 2818 (2000): "HTTP Over TLS".

[10]	Void

[11]	Void

[12]	Void

[13]	3GPP TS 33.210: "3G Security; Network Domain Security; IP network layer security".

[14]	Void.

[15]	 Void.

[16]	3GPP TS 33.221: "Generic Authentication Architecture (GAA); Support for subscriber certificates".

[17]	Void.

[18]	3GPP TS 24.109: "Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[19]	3GPP TS 29.109: "Generic Authentication Architecture (GAA), Zh and Zn Interface based on the Diameter protocol; Stage 3".

[20]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[21]	Void.

[22]	 Void.

[23]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[24]	W3C Working Draft (Jan 22, 2013): "HTML5.1 Nightly – A vocabulary and associated APIs for HTML and XHTML", work in progress, http://dev.w3.org/html5/spec/.

[25]	IETF RFC 5929 (2010): "Channel Bindings for TLS".

[26]	W3C Working Draft (Oct 20, 2011): "File API", work in progress, http://www.w3.org/TR/FileAPI/.

[27]	W3C Candidate Recommendation (Dec 8, 2011): "Web Storage", work in progress, http://www.w3.org/TR/webstorage/

[28]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[29]	IETF RFC 5705 (2010): "Keying Material Exporters for Transport Layer Security (TLS)".

[30]	IETF RFC 8446 (2018): "The Transport Layer Security (TLS) Protocol Version 1.3".

[31]	IETF RFC 7235: "Hypertext Transfer Protocol (HTTP/1.1): Authentication".

[32]	IETF RFC 7616: "HTTP Digest Access Authentication".

[33]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[xx]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)"

[xy]				IETF RFC 9202: "Datagram Transport Layer Security (DTLS) Profile for Authentication and 							Authorization for Constrained Environments (ACE)"

**** End of Changes****