3GPP TSG-SA3 Meeting #110 	S3-230899

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.222 | CR | draftCR | rev | <-> | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Shared key-based UE authentication with certificate-based NAF authentication to TS 33.222 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA_GBA_DTLS |  |  |  |  |  | Date: |  |  | 2023-2-6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In order to complete the work on the DTLS for GBA Ua protocol, it is required to specify the DTLS profile for Shared key-based UE authentication with certificate-based NAF authentication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add DTLS profile for Shared key-based UE authentication with certificate-based NAF authentication |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete work for the DTLS for GBA Ua protocol |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of Change 1****

Annex X (normative): 
DTLS profiles for  GBA Ua

X.1	DTLS based protocols

X.1.1	General

This Annex covers the aspects specific to the DTLS profiles for GBA Ua.

X.1.2	Shared key-based UE authentication with certificate-based AF authentication

The authentication mechanism described in this section for ME-based application is mandatory to implement in ME and optional to implement in NAF.

The authentication mechanism described in this section for UICC-based application is mandatory to implement in the UICC and optional to implement in the NAF.

This section explains how the procedures specified in TS 33.220 [3] have to be enhanced when CoAP and DTLS is used between a ME and a NAF or between the UICC and the NAF.

1)	When the CoAP client starts communication via Ua reference point with the NAF, it shall establish a DTLS tunnel with the NAF. The NAF is authenticated to the CoAP client by means of a public key certificate. The CoAP client shall verify that the server certificate corresponds to the FQDN of the NAF it established the tunnel with. No client authentication is performed as part of DTLS (no client certificate necessary).

2)	The CoAP client sends an CoAP request to the NAF inside the DTLS tunnel. The CoAP client shall indicate to the NAF that GBA-based authentication is supported by adding one or more constant string(s) to the  payload. If the client supports AKA-based authentication the constant string added shall be either "3gpp-gba" for ME-based applications or "3gpp-gba-uicc" for UICC based applications. Or, if the client supports GBA_Digest it shall add the constant string “3gpp-gba-digest” The UE shall send the hostname of the NAF in "Option" field.

3)	In response to the CoAP request received from CoAP client over the Ua reference point,  The NAF first verifies that the payload received in the CoAP request in step 2, if any, indicates a GBA mode acceptable to the NAF. If so, the NAF selects one acceptable GBA mode and includes the corresponding constant string "3GPP-bootstrapping" (in case "3gpp-gba" is the selected GBA mode ), or "3GPP-bootstrapping-uicc" (in case "3gpp-gba-uicc" is the selected GBA mode ), or "3GPP-bootstrapping-digest" (in case "3gpp-gba-digest" is the selected GBA mode),  and the FQDN of the NAF (for all cases) in the payload field.  In the selection of the GBA mode by the NAF, AKA-based modes shall take priority over GBA_Digest.

If the NAF has been configured to forbid the access to the service for all indicated GBA modes or if the NAF does not support any of the indicated GBA modes, then the NAF shall respond with the appropriate error code and terminate the DTLS connection with the UE.

4)	On receipt of the response from the NAF, the CoAP client shall verify that the FQDN in the payload field corresponds to the FQDN of the NAF it established the DTLS connection with. On failure the CoAP client shall terminate the DTLS connection with the NAF.Furthermore, if no key Ks corresponding to the GBA mode indicated by the NAF in step 3 is available in the UE the UE shall perform a run of the Ub protocol with the BSF for the corresponding GBA mode. If no such key Ks can be obtained the UE shall abandon the GBA-based authentication.

5)	In the following request to NAF the CoAP client sends a message with POST method. And in the payload field, B-TID as key id.The NAF-specific key (Ks_(ext)_NAF in the case of ME-based application (including GBA_Digest) or Ks_int_NAF in the case of UICC-based application) is used as the key to compute an authentication code.

If CBOR  is used, IETF RFC 8152[xx] specified how to compute and verify the MAC.

6)	On receipt of this request the NAF shall get the NAF-specific key (Ks_(ext)_NAF or Ks_int_NAF) from BSF over Zn using the B-TID received as user name attribute in the query. And the NAF shall verify the authentication code received from step 5.

7)	After the completion of step 6), UE and NAF are mutually authenticated as the DTLS tunnel endpoints.

X.1.3	Shared key-based mutual authentication between UE and AF

**** End of Change 1****

**** Start of Change 2****

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.002: "Network architecture".

[2]	3GPP TS 22.250: "IP Multimedia Subsystem (IMS) group management"; Stage 1".

[3]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture".

[4]	Void

[5]	3GPP TS 33.141: "Presence Service; Security".

[6]	Void.

[7]	Void.

[8]	Void.

[9]	IETF RFC 2818 (2000): "HTTP Over TLS".

[10]	Void

[11]	Void

[12]	Void

[13]	3GPP TS 33.210: "3G Security; Network Domain Security; IP network layer security".

[14]	Void.

[15]	 Void.

[16]	3GPP TS 33.221: "Generic Authentication Architecture (GAA); Support for subscriber certificates".

[17]	Void.

[18]	3GPP TS 24.109: "Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[19]	3GPP TS 29.109: "Generic Authentication Architecture (GAA), Zh and Zn Interface based on the Diameter protocol; Stage 3".

[20]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[21]	Void.

[22]	 Void.

[23]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[24]	W3C Working Draft (Jan 22, 2013): "HTML5.1 Nightly – A vocabulary and associated APIs for HTML and XHTML", work in progress, http://dev.w3.org/html5/spec/.

[25]	IETF RFC 5929 (2010): "Channel Bindings for TLS".

[26]	W3C Working Draft (Oct 20, 2011): "File API", work in progress, http://www.w3.org/TR/FileAPI/.

[27]	W3C Candidate Recommendation (Dec 8, 2011): "Web Storage", work in progress, http://www.w3.org/TR/webstorage/

[28]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[29]	IETF RFC 5705 (2010): "Keying Material Exporters for Transport Layer Security (TLS)".

[30]	IETF RFC 8446 (2018): "The Transport Layer Security (TLS) Protocol Version 1.3".

[31]	IETF RFC 7235: "Hypertext Transfer Protocol (HTTP/1.1): Authentication".

[32]	IETF RFC 7616: "HTTP Digest Access Authentication".

[33]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[xx]	IETF RFC 8152: "CBOR Object Signing and Encryption (COSE)"

**** End of Change 1****