# 3GPP TSG-SA3 Meeting #110 	S3-230911

# Athens, Greece, 20 - 24 February 2023

# Source:	ZTE

# Title:	updates to the solution 15

# Document for:	Approval

# Agenda Item:	5.6

1	Decision/action requested

This contribution proposes to update the solution 15 of the TR 33.737.

2	References

[1] 	TR33.737 v0.5.0

3	Rationale

The AUSF/UDM cannot store the accurate UPF of the UE in the visited network, but the AUSF/UDM can find the AMF of UE in the visited network base on the SUPI of UE.

It is proposed to update to the solution.

4	Detailed proposal

********************Start of 1st Change******************

6.15	Solution #15: AKMA roaming for external AF in Data Network

6.15.1	Introduction

This solution addresses the KI #1.

This solution considers external AF in a data network in roaming scenarios. Specifically, AKMA architecture in clause 6.3 TS 33.535 [1] is reused to support the AKMA roaming which involves external AF in a data network.

6.15.2	Solution details

To support AKMA roaming which involves external AF in a data network, this solution reuses clause 6.3 TS 33.535 with the following modifications.

- 	In step 3, the NEF checks whether the connected AF is an external AF in the data network based on the local policies. If the AF is an external AF in the data network, the NEF sends an AF indicator to the AAnF in the home network of UE. The AF indicator indicates that the AF is an external AF in a data network.

-	If the AAnF recieves the AF indicator and the UE is roaming, the AAnF in the home network sends SUPI of the UE, AF_ID, KAF, the KAF expiration time (KAF exptime) to the visited network (e.g., AMF in the visited network) of the UE. The hAAnF may obtain the visited network name of the UE from the AUSF/UDM base on SUPI of the UE.

6.15.3	Solution Evaluation

This solution address part of case 2(UE in VPLMN accessing an internal AF of the VPLMN) and case 3 (UE is in VPLMN and accessing an external AF in the Data Network)

This solution has impact to both hAAnF and NEF. In specific, NEF needs to send an AF indicator to the AAnF in the home network of UE based on local policies. The AF indicator indicates that the AF is an external AF in a data network.

The advantages of this solution is given as follows.

In specific, this solution enables hAAnF to differ the VPLMN AF and the external AF.

For external AF, hAAnF needs to send AKMA related information (e.g., KAF) to the VPLMN.

For VPLMN AF, hAAnF does not need to send AKMA related information (e.g., SUPI of the UE, AF_ID, KAF, the KAF expiration time) to the VPLMN.

The disadvantages of this solution is given as follows.

This solution has impacts to both AAnF and NEF.

********************End of 1st Change******************