# 3GPP TSG-SA3 Meeting #110	S3-230940

# Athens, Greece, 20th – 24th February, 2023

# Source:	OPPO

# Title:	Add Evaluation in Sol #6

# Document for:	Approval

Agenda Item:	5.3

1	Decision/action requested

This pCR proposes to add evaluation for Sol #6 of TR 33.740.

2	References

N/A

3	Rationale

This solution proposes to add evaluation for solution#6 of TR 33.740.

4	Detailed proposal

********* Begin 1st change*********

### 6.6.3	Evaluation

The solution fulfils the security requirement of Key Issue #2: Security of UE-to-UE Relay.

The solution supports security per hop links (i.e. PC5 link between Source UE and UE-to-UE Relay, as well as between UE-to-UE Relay and Target UE) and security E2E PC5 link between source UE and target UE setup.

The solution supports security method (i.e. Out-of-Coverage). With the provisioned/pre-configured security materials, the security keys can be generated without the assistance of the network.

This solution is suitable for the standalone discovery.

This solution proposes that the security credential and security credential ID could be pre-configured in the ProSe enabled UE (i.e. Source UE, Target UE and UE-to-UE Relay), and provisioning of the security credential and the security ID in the ProSe enabled UE is out of the 3GPP scope.

********* End 1st change*********