3GPP TSG-SA3 Meeting #110 	S3-230958

# Athens, Greece, 20 - 24 February 2023											(revision of S3-yyxxxx)

# Source: 	Huawei, HiSilicon

# Title:	Update conclusion on KI#3

# Document for:	Approval

Agenda Item:	5.8

1	Decision/action requested

It is requested to approve this proposal in TR 33.738.

2	References

[1]   3GPP TR 33.738 V0.3.0 "Study on security aspects of enablers for Network Automation for 5G - phase 3"

3	Rationale

NA

4	Detailed proposal

*** 1st CHANGE ***

7.3	Conclusion on Key Issue #3 "Security for AI/ML model storage and sharing"

The conclusions for KI#3 are:

-	Authorization of the model retrieval at the NRF uses OAuth 2.0 token-based authorization. The NRF uses information provided by the MTLF.

Editor's Note : The requirement for NF service consumers (NFc) to be registered in the NRF as part of the authorization procedure is ffs.

Editor's Note : The requirement for the AI/ML model to be stored in encrypted format and corresponding key management aspects are ffs.

-	As per the request of Analytics Id by the NFc, the MTLF performs authorization of the corresponding model retrieval per selected model. If one-time URL is used, the MTLF may return ADRF ID along with a token for accessing ADRF. The token is requested by the MTLF and is generated by the NRF.

NOTE: the model delivery procedure is to be defined by 3GPP SA2.

- If one-time URL is used, the AnLF may retrieve a token to access ADRF from the NRF. The NRF may check with MTLF for authorization of the corresponding model retrieval per selected model.

Editor's Note: The granularity of the authorization at the MTLF is ffs.

-	ADRF verifies that the requested AI/ML model can be retrieved by the NF consumer(s) (MTLF or AnLF). If one-time URL is used, the ADRF sends one-time URL to the AnLF for secure AI/ML model retrieval after successful verification.

*** END OF 1st CHANGE***