3GPP TSG-SA3 Meeting #110 	S3-230978

# Athens, Greece, 20 - 24 February 2023											(revision of S3-yyxxxx)

# Source:	Huawei, HiSilicon

# Title:	Home network triggered primary authentication procedure

# Document for:	Approval

Agenda Item:	4.14

1	Decision/action requested

to approve the procedure of HONTRA.

2	References

NA

3	Rationale

Based on the discussion in SA3#109 meeting and the agreement of the HONTRA, a high-level procedure is needed to be specified. This contribution brings the high-level procedure.

4	Detailed proposal

It is proposed to approve the following new procedure.

*** BEGIN of 1st CHANGE ***

6.X	Home network triggered primary authentication procedure

6.X.1	General

In case the HN and the SN support the services defined in clause 14.2.x and 14.x.1, then the UDM in the HN is able to trigger a primary authentication procedure as described in the following clause.

6.X.2	Security mechanisms

When the UDM supports the services specified in clause 14.2.x, then the UDM is able to trigger a primary authentication at any time based on a request from another NF or based on a local operator policy.

Figure 6.x.2-1 Home Network triggered primary authentication procedure

0a.[optional] The UDM may be pre-configured with a local policy in order to determine when to trigger a primary authentication procedure.

0b. [optional]	NF may send a Nudm_HNAuthentication_Authenticate request message to the UDM including the SUPI in order to trigger a primary authentication procedure run for the target UE (identified by the SUPI).

NOTE: The NF could be the AUSF and the condition is the SoR Counter wrap around, or the NF could be the AAnF and the condition is the KAKMA defined in TS 33.535[91] needs to be refreshed.

1. Based on the received request or the local policy, if the UDM can trigger the primary authentication, the AMF/SEAF identifies the serving AMF/SEAF of the target UE. Otherwise, the UDM directly goes to step5 with a failure cause.

2. The UDM sends a Namf_HNAuthentication_Authenticate Request message to the AMF/SEAF with the UE’s SUPI.

3. Based on the request from the UDM, if the AMF/SEAF can start to run the primary authentication procedure as described in steps 4, the AMF/SEAF sends the Namf_HNAuthentication_Authenticate Response message to the AMF/SEAF with a successful indication. Otherwise, the Namf_HNAuthentication_Authenticate Response message contains a failure cause.

4. The AMF/SEAF starts the primary authentication procedure as defined in clause 6.2.1 of the present document.

5. The UDM may send the Nudm_HNAuthentication_Authenticate Response message to the UE contains a Result indication. If the primary authentication for the UE runs successfully, the Result indicates the UE has been authenticated successfully. Otherwise, the message contains a failure cause that may indicate the UDM or the AMF/SEAF cannot trigger the procedure or the UE does not pass the authentication.

*** END of CHANGE ***