3GPP TSG-SA3 Meeting #110 	S3-230982

# Athens, Greece, 20 - 24 February 2023											(revision of S3-yyxxxx)

# Source:	Huawei, HiSilicon

# Title:	General description about secondary authentication

# Document for:	Approval

Agenda Item:	4.10

1	Decision/action requested

To approve the procedure and the procedure can be merged into the living doc.

2	References

NA

3	Rationale

This contribtion proposes Secondary Authentication Procedure for Remote UE without N3IWF based on the S3-222400.

4	Detailed proposal

It is proposed to approve the following new procedure and the content can be merged to the living doc.

*** BEGIN of 1st CHANGE ***

6.X.Y	Secondary Authentication Procedure for Remote UE

6.X.Y.1 General

This clause specifies the 5G Prose Remote UE specific secondary authentication between a 5G ProSe Remote UE, which is different from the secondary authentication defined in TS 33.501 [3], via a 5G ProSe Layer-3 UE-to-Network Relay without N3IWF and an external Data Network (DN) based on network-controlled authorization (i.e. using 5G ProSe Remote UE specific authentication) as described in clause 6.3.3.3.2. This procedure is optional to support.

The SMF of the 5G ProSe UE-to-Network Relay triggers the secondary authentication of the 5G ProSe Remote UE based on the subscription information and the local configuration of the SMF when it receives a NAS message (e.g. Remote UE Report) from the 5G ProSe UE-to-Network Relay.

The EAP framework specified in IETF RFC 3748 [12] shall be used for authentication between the 5G ProSe Remote UE and a DN-AAA server in the external data network.

Following clause describes the procedures for initial secondary authentication of the 5G ProSe Remote UE with the external DN-AAA server.

*** END of CHANGE ***