3GPP TSG-SA3 Meeting #110 	S3-23xxxx

# Athens, Greece, 20 - 24 February 2023											(revision of S3-yyxxxx)

# Source:	Huawei, HiSilicon

# Title:	Update solution #6

# Document for:	Approval

Agenda Item:	5.13

1	Decision/action requested

Approve the changes of solution details in clause 6.6.2.1 and the changes of evaluation in clause 6.6.3

2	References

[1]	3GPP TR 33.887 Study on Security aspects for 5WWC Phase 2

3	Rationale

Reauth Id is used for identifying UE. The Reauth Id transferred between UE and TNGF is not integrity protected, and there also lacks authentication of UE when UE moves to a new TNAP, thus there needs a MAC added in EAP-request of step5-6 for data protection and UE authentication.

There is a EAP-Req/5G-notification in step9, but the corresponding EAP-Res/5G-notification is missing.

4	Detailed proposal

**** START OF CHANGE ****

6.6	Solution #6: TNAP mobility solution with count

6.6.1	Introduction

This solution addresses the security issue due to  TNAP mobility defined in KI#4.

6.6.2	Solution details

#### 6.6.2.1	Procedure

Figure 6.6.2-1 TNAP mobility procedure

- UE is connected to TNAP#1 via the procedure defined in TS 33.501 figure 7A.2.1-1. Once authenticated, TNGF sends the reauth Id to UE over the protected interface. Reauth Id can be a generated as <PLMNID><TNGF_ID> <Temp Id>.

Note: TNGF Id could be TNGF address (like fqdn)already defined by SA2.

2, 3.	UE decides to move from TNAP#1 to TNAP#2 and creates an L2 connection with TNAP#2.

4, 5, 6.		TNAP#2 sends the L2 EAP-Request for Identity towards the UE and the UE responds back with an L2 EAP-Response with Identity and a TNAP_Mobility_Indication flag and a MAC value. The MAC value is calculated with the input of Reauth ID and TNAP_Mobility_Indication. The TNAP2 forwards the EAP response encapsulated into AAA message with reauth Id and the TNAP_Mobility_Indication flag and the MAC towards TNGF.

7,8. 	 Based on the reauth Id, TNFG identifies the UE and retrieves the context and  checks if the stored context in step 1 is valid and then derives the TNAP’ keys as described in section 6.6.2.2 of this document. The TNGF responds back to TNAP#2 with the generated key. In TNAP#2, the newly received TNAP key is considered as Pairwise Master Key (PMK).

9, 10,11.	The TNAP#2 sends an EAP-notification to the UE with the update flag. Based on the flag, UE updates the counter and derives the keys. UE sends an EAP-Res/5G-notificationto the TNAP#2. A 4-way handshake is executed (see IEEE 802.11) which establishes a security context between the WLAN AP and the UE that is used to protect unicast and multicast traffic over the air.

Once the procedure is complete, the TNGF sends the new reauth Id to UE over the secure interface that UE can use for the next interaction.

Editor's Notes: How to prevent replay attacks needs further study

NOTE: If the UE gets the new IP configurations from TNAP2, then the UE updates the SA address using IKE informational request "UPDATE_SA_ADDRESS" to TNGF for further communications.

#### 6.6.2.2	Key derivation

Derivation of KTNAP' from KTNGF during mobility use the following input parameters.

-	FC = 0xWX

-	P1 = COUNT,

-	L1 = length of COUNT (i.e. 0x00 0x04)

The input key KEY shall be KTNGF.

In this case, the COUNT shall be the downlink NAS COUNT of the Non-3GPP access.

6.6.3	Evaluation

TNGF and UE: Generate new keys based on the count value and handle the reauth Id.

TNGF: Authenticate the UE by verifying the MAC value of reauth ID in step7.

TNAP: use updated keys for the mobility scenario

UE: Integrity protection of the Reauth ID.

Editor's Notes: The count desynchronization issue and further evaluation are FFS.

**** END OF CHANGE ****