3GPP TSG-SA3 Meeting #110	S3-231029

# Athens, Greece, 20 - 24 February 2023

# Source:	Huawei, HiSilicon

# Title:	New solution for protecting Ranging unicast communication

# Document for:	Approval

Agenda Item:	5.19

1	Decision/action requested

Approve the new solution proposal to TR 33.893

2	References

N/A

3	Rationale

The contribution proposes to add a new solution to provide secure direct communications.

4	Detailed proposal

***BEGINNING OF THE 1st CHANGE***

6.X	Solution #X: Security of Ranging unicast communication

6.X.1	Introduction

This solution addresses the protection of direct communication (unicast) for Ranging/SL Positioning service as specified in Key Issue #4. Generally, the solution uses the direct communication security defined for 5G ProSe in TS 33.503 [6] and/or for 5G V2X in TS 33.536 [5] as a baseline and adjusts to the Ranging/SL Positioning scenario.

6.X.2	Solution details

In this solution, the Ranging/SL positioning capable UEs are provisioned with the list of Ranging/SL Positioning services (identified by using V2X service type or ProSe identifier), with Geographical Area(s) and their PC5 security policies. The definition of the PC5 security policies follows the definition in 5G eV2X in TS 33.536 [5] and/or in ProSe in TS 33.503 [6].

During the establishment of the direct communication for the Ranging/SL positioning service, the UEs use the PC5 security policy to negotiate the final security protection status, as defined in 5G ProSe in TS 33.503 [6] and/or in 5G V2X in TS 33.536 [5]. The Ranging/SL Positioning signalling are protected based on the security requirement from the Ranging/SL Positioning service, which is reflected by the PC5 UP security policies.

6.X.3	Evaluation

This solution addresses the first three security requirements of Key Issue #4. The mutual authentication between two UEs during one-to-one communication is supported by reusing the PC5 security establishment procedure in 5G ProSe in TS 33.503 [6] and/or in 5G V2X in TS 33.536 [5].

***END OF THE CHANGES***