3GPP TSG-SA3 Meeting #110	  S3-231052

February 20 - 24, 2023, Athens, Greece

Title:	[Draft] Reply LS on security for selective SCG activation

Response to:	S3-230618

Release:	Rel-18

Work Item:	NR_mob_enh2-Core

Source:	OPPO [to be SA3]

To:	RAN2

Cc:	-

Contact person:

Name:	Lihui Xiong

E-mail Address:	xionglihui at oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall description

SA3 thanks RAN2 for their LS on security for selective SCG activation.

From SA3 point of view, the proposed handling of sk-counter/ S-KgNB in these scenarios described in this liaison is not acceptable. When the UE changes from SN #1 to SN #2, the sk-counter/ S-KgNB is updated. After then, the same sk-counter/ S-KgNB should not be used while UE move back to SN #1. Reusing the same sk-counter/ S-KgNB may increase the risk of key leakage.

The security requirement needs to be kindly considered: When the UE changes the serving SN, i.e., the UE radio bearer(s) is moved from the SN, the SN and UE shall delete the S-KgNB. The sk-counter/ S-KgNB should be updated after the UE changes serving SN. And the same sk-counter should not be used to derived the same S-KgNB if UE changes back to the same SN after moving to another SN.

- Actions

To RAN2

ACTION: 	SA3 kindly asks RAN2 to take the above feedback into account.

- Dates of next TSG SA WG 3 meetings

SA3#110-bis-e	17 - 21 April 2023	Online

SA3#111			22 – 26 May, 2023	EU, EU