3GPP TSG-SA3 Meeting #110 	S3-23xxxx

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | <CR#> | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to security aspects of eNA Service in 33.501 for Rel-18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 930007 |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CT4 asks SA3 to consider aligning stage 2 on stage 3: - -the (Rel-15 onwards) nfInstanceId IE in the Access Token Request and the subject claim in the access token to indicate the NF instance ID of the intermediate NF service consumer (e.g. DCCF) creating the subscription at the NF service producer and - -the (Rel-17 onwards) sourceNfInstanceId IE in the Access Token Request and access token claims to indicate the NF instance ID of the source NF instance ID (e.g. NWDAF). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aligning stage 2 on stage 3 on Authorization of NF service consumers (e.g. NWDAF) for data access via DCCF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignments between requirements specified in TS 29.510 and in Annex X of TS 33.501 on Authorization of NF service consumers (e.g. NWDAF) for data access via DCCF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# X.2	Authorization of NF Service Consumers for data access via DCCF

The detailed procedure for NF Service Consumer to receive data from Service Producers via DCCF is depicted in Figure X.2-1:

Figure X.2-1: NF Service Consumer Authorization to receive data from NF Service Producers via DCCF

1-3. NF Service Consumer shall send a request to the NRF to receive an access token to request services of DCCF, to be used for data collection request. NRF after verifying shall generate access token and sends it to the NF Service Consumer.

4.	The NF Service Consumer initiates a NF service request to the DCCF which includes the access_token_nwdaf. The NF Service Consumer shall also generate a Client Credentials Assertion (CCA) token (CCA_NWDAF) as described in the clause 13.3.8 and include it in the request message in order to authenticate itself towards the NF Service Producers.

NOTE 0: The procedure of NF Service Consumer (e.g. NWDAF) requesting the services provided by NF Service Producer via DCCF is defined in Clause 6.2.6.3 of TS 23.288 [105].

5.	The DCCF shall verify if the access_token_nwdaf is valid and executes the service.

6.	The DCCF determines the NF Service Producer(s) from where the data is to be collected (as specified in Clause 6.2.6.3.2 in TS 23.288[105]).

NOTE 1: 	If the NF Service Consumer sends the NF Service Producer information (i.e. NF Service Producer type and Instance ID) along with the service request in Step 4, then DCCF does not determine the NF Service Producer, but requests an access token from the NRF using the NF Service Producer details sent by the NF Service Consumer (as described in Step 7.)

7.	The DCCF sends a Nnrf_AccessToken_Get request to NRF including the information to identify the target NF (NF Service Producer), the source NF (NF Service Consumer e.g. NWDAF), the NF Instance ID of DCCF and the CCA_NWDAF provided by the NF Service Consumer.

NOTE 1a: The NF Instance ID of the source NF is included in a new IE and NRF from Rel-16 or earlier will ignore the new IE.

8.	The NRF shall check whether the DCCF and the NF Service Consumer (e.g. NWDAF) are allowed to access the service provided by the identified NF Service Producers , and the DCCF as the proxy is allowed to request the service from the identified NF Service Producers on behalf the NF Service Consumer. NRF authenticates both DCCF and NWDAF based on one of the SBA methods described in clause 13.3.1.2. DCCF may include an additional CCA for authentication.

NOTE 2: A Rel-16 NRF takes CCA to authenticate NF Service Consumer if available (i.e., authentication is not based on TLS).

NOTE 3: In the case the NRF is from Rel-16 or earlier, after the NRF receives Nnrf_AccessToken_Get request, the NRF validates whether the DCCF is authorized to receive the requested service from the NF Service Producer. The NRF from Rel-16 or earlier does not validate whether the NF Service Consumer (e.g., NWDAF) is authorized to receive the requested service.

9.	The NRF after successful verification then generates and provides an access token to the DCCF as described in the clause 13.4.1.1.2, with DCCF Instance (subject), and an additional access token claim containing the identity of NF Service Consumer, in order to authorize both NF Service Consumer (e.g.. NWDAF) and DCCF to consume the services of NF Service Producer.

NOTE 4: In the case the NRF is from Rel-16 or earlier, the NRF generates an OAuth2.0 access token with “subject” claim mapped to the DCCF and no additional claim for the NF Service Consumer (e.g., NWDAF) identity is added.

10.	The DCCF requests service from the NF Service Producer. The request also consists of CCA_NWDAF, so that the NF Service Producer(s) authenticates the NF Service Consumer (e.g. NWDAF).

11.	The NF Service Producer(s) authenticate the NF Service Consumer and verify the access token as specified in the Clause 13.4.1.1.2. After authentication and authorization is successful, the NF Service Producer(s) assure that the DCCF as the proxy is allowed to receive the response message on behalf the NF Service Consumer, and execute the service after successful verification. DCCF may include an additional CCA for authentication.

NOTE 5: Rel-16 NF Service Producer takes CCA to authenticate NF Service Consumer if available (i.e., authentication is not based on TLS).

12. The NF Service Producer(s) shall provide requested data to the DCCF.

13. The DCCF forwards the received data to the NF Service Consumer(s).

NOTE 6: In the case a new NF Service Consumer comes at a later stage to request the data, which is already being collected by DCCF, steps 1-10 apply. When the request is received by the NF Service Producer (i.e. the data producer), it authenticates the NF Service Consumer and verifies the access token provided along with the service request and sends to DCCF the access token verification response. DCCF based upon the response received, either updates the subscription info to include the newNF Service Consumer as well and sends the data to both the consumers (as specified in Clause 6.2.6.3.2 in TS 23.288 [105]), or in the case of access token verification failure, the DCCF rejects the request received by the NF Service Consumer.

NOTE 7: In the case the NF Service Producer is from Rel 16 or earlier, the NF Service Producer authorizes DCCF by validating the received OAuth2.0 access token which has the "subject" claim maps to the DCCF. Rel-16 or earlier NF Service Producer authorization of the the NF Service Consumer (e.g., NWDAF) is a deployment specific based on any of the available 5GC authorization method(s).