3GPP TSG-SA3 Meeting #110 	S3-231119

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1568 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add restriction for multi registrations in two PLMNs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1.Parallel registration from UE results in KAUSF mis-synchronization issue between UE and HN(AUSF) and results in failure (integrity check failure always) for the UPU and SoR procedures. 2. Parallel registration from UE results in AKMA context mis-synchronization issue between UE and AAnF and results in “no possibility” of Application session establishment between UE and AF in Ua* interface. 3. two parallel EAP sessions for same EAP ID cannot be supported based on EAP protocol. More detailed discussion paper presented S3-223807 for reference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update rules related to parallel NAS connections to control sequence also for multi-registration in different PLMNs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KAUSF used in UE and network side may not be synchronized, that may cause failure on other procedures like UPU, SoR, AKMA, etc. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

################### start the 1st change ##################

#### 6.9.5.2	Rules related to parallel NAS connections

Concurrent runs of security procedures in parallel over two different NAS connections when terminated in the same or different AMF(s) can lead to race conditions and mismatches between the security contexts in the network and the UE. In order to avoid such mismatches, the following rules shall be followed:

- The SEAF/AMF shall not initiate a primary authentication or NAS SMC procedure in case a primary authentication or a NAS SMC procedure is ongoing on a parallel NAS connection. Authentication procedures followed by a NAS SMC procedures taking the new 5G security context into use, shall be performed on one NAS signalling connection at a time.

NOTE: This rule is only applied on the case that two different NAS connections terminated in the same AMF.

2.	When the AMF has sent a NAS Security Mode Command to a UE in order to take a new KAMF into use and receives a context transfer request message for the UE from another AMF, the AMF shall wait for the completion of the NAS SMC procedure (e.g. receiving NAS Security Mode Complete) before transferring the context.

NOTE: This rule is only applied on the case that two different NAS connections terminated in the same AMF.

3.	The UE shall not initiate a NAS registration over a second NAS connection to an AMF of the same or different network(s) before primary authentication on the first NAS connection is complete.

###################  end of the change  ##################