3GPP TSG-SA3 Meeting #110 	S3-231145

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.402 | CR | 0148 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Aligning DNS and ICMP security for non-3GPP access with 3GPP access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The proposed changes in this CR were endorsed at SA3#109 as S3-223685 (CR 0147r0). Annex L of TS 33.401 has the following recommendation for DNS and ICMP security when using 3GPP access to EPC: "The security measures specified in Annex P of TS 33.501 can be used to protect the DNS and ICMP messages that are carried over the user plane." Annex P of TS 33.501 applies to both 3GPP and non-3GPP access to 5GCN. However, for EPC, the statement in TS 33.401 only applies to 3GPP access, since the scope of TS 33.401 is 3GPP access. UEs move between 3GPP access and non-3GPP access, and between EPS and 5GS. It does not seem reasonable that DNS and ICMP traffic that is protected over 3GPP access to EPC and 5GCN, and over non-3GPP access to 5GCN, should be left unprotected when the UE moves to non-3GPP access to EPC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of the statement that security measures in Annex P of TS 33.501 can be used to protect the DNS and ICMP messages that are carried over the user plane, in a new informative annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DNS and ICMP traffic that is protected over 3GPP access to EPC and 5GCN, and over non-3GPP access to 5GCN, could be left unprotected when the UE moves to non-3GPP access to EPC. It could also lead to complications for network and UE configurations if non-3GPP access to EPS needs to be handled differently than 3GPP access to EPS and 5GS, and non-3GPP access to 5GS, as e.g.: - when a UE transfers a PDN connection from non-3GPP access to EPC to 3GPP access to EPC (or interworks a PDN connection in non-3GPP access to EPC to a PDU session in 5GS), the UE needs to start using (D)TLS for protecting DNS communication; and - when a UE transfers a PDN connection from 3GPP access to EPC to non-3GPP access to EPC (or interworks a PDU session in 5GS to a PDN connection in non-3GPP access to EPC), the UE needs to stop using (D)TLS for protecting DNS communication. Given that the same DNS servers in the same packet data network are reached via non-3GPP access to EPC, 3GPP access to EPC and 5GS, such activation and de-activaction of (D)TLS is an unnecessary complexity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	IETF RFC 4877: "Mobile IPv6 Operation with IKEv2 and the Revised IPsec Architecture".

[3]	Void.

[4]	IETF RFC 5778: "Diameter Mobile IPv6: Support for Home Agent to Diameter Server Interaction".

[5]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[6]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[7]	IETF RFC 4187 (January 2006): "Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA)".

[8]	3GPP TS 23.003: "Numbering, addressing and identification".

[9]	3GPP TS 33.234: "3G: security; Wireless Local Area Network (WLAN) interworking security" (Release 12).

[10]	IETF RFC 4072 (August 2005): "Diameter Extensible Authentication Protocol (EAP) Application".

[11] 	3GPP TS 33.102: "3G security; Security architecture".

[12]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[13]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[14]	3GPP TS 23.203: "Policy and charging control architecture".

[15]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Overall description; Stage 2".

[16]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security Architecture".

[17]	IETF RFC 3344: "IP Mobility Support for IPv4".

[18]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[19]	IETF RFC 5295: "Specification for the Derivation of Root Keys from an Extended Master Session Key (EMSK)".

[20]	3GPPP TS 24.303: "Mobility Management based on Dual-Stack Mobile IPv6; Stage 3".

[21]	IETF RFC 4433: "Mobile IPv4 Dynamic Home Agent (HA) Assignment".

[22]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3  ".

[23]	IETF RFC 5448: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA') ".

[24]	3GPP TS 33.222: "Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)".

[25]	3GPP TS 29.109: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Generic Authentication Architecture (GAA); Zh and Zn Interfaces based on the Diameter protocol; Stage 3".

[26] - [28]	Void.

[29]	3GPP TS 33.223: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) Push function".

[30]	IETF RFC 5996: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[31]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[32]	3GPP TS 29.275: "Proxy Mobile IPv6 (PMIPv6) based Mobility and Tunnelling protocols; Stage 3".

[33]	IETF RFC 4739: "Multiple Authentication Exchanges in the Internet Key Exchange (IKEv2) Protocol".

[34]	3GPP TS 33.203: "Access security for IP-based services".

[35]	IETF RFC 3948: "UDP Encapsulation of IPsec ESP Packets".

[36]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[37]	IETF RFC 6347: "Datagram Transport Layer Security Version 1.2".

[38]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[39]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[40]	Federal Information Processing Standard (FIPS) draft standard: "Advanced Encryption Standard (AES)", November 2001.

[41]	RFC 1421, February 1993: "Privacy Enhancement for Internet Electronic Mail: Part I: Message Encryption and Authentication Procedures".

[42]	Open Mobile Alliance OMA-WAP-OCSP V1.0: "Online Certificate Status Protocol Mobile Profile". URL: http://www.openmobilealliance.org/

[43]	IETF RFC 4806, Februari 2007: Online Certificate Status Protocol ("OCS)P Extensions to IKEv2".

[44]	IETF RFC 4282, December 2005: "The Network Access Identifier", (Obsoletes RFC2486)

[45]	IETF RFC 2865, June 2000: "Remote Authentication Dial In User Service (RADIUS)".

[46]	IETF RFC 6696: "EAP Extensions for the EAP Re-authentication Protocol (ERP)".

[47]	IETF RFC 6942: "Diameter Support for the EAP Re-authentication Protocol (ERP)".

[48]	3GPP TS 24.502: "Access to the 3GPP 5G System (5GS) via non-3GPP access networks; Stage 3".

[xx]	3GPP TS 33.501: "Security architecture and procedures for 5G System"

*** NEXT CHANGES ***

Annex X (informative):
Security Aspects of DNS and ICMP

# X.1	General

The security measures specified in Annex P of TS 33.501 [xx] can be used to protect the DNS and ICMP messages that are carried over the user plane.

*** END CHANGES ***