3GPP TSG-SA3 Meeting #110 	S3-231149

# Athens, Greece, 20 - 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1573 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SEPP to include and verify the source PLMN-ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson [was: Ericsson, Mavenir, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In a roaming scenario where one SEPP serves a given PLMN, and such PLMN has multiple PLMN-IDs but uses the same N32 connection for all PLMN-IDs, the current specification does not describe the solution how to identify and verify the source PLMN-ID of a message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For an NF supporting multiple PLMN-IDs, the sending NF shall include the PLMN ID for the HTTP request. The sending SEPP shall include the source PLMN ID (i.e., 3gpp-Sbi-Originating-Network-Id) in the signaling message. The receiving SEPP shall discard messages if the source PLMN ID (i.e., 3gpp-Sbi-Originating-Network-Id) included in the signaling message does not belong to the sending SEPP’s own PLMN. In case the NF has not included the 3gpp-Sbi-Originating-Network-Id in the signaling message, and the sending SEPP represents multiple PLMN-IDs, it is up to configuration and deployment to determine which PLMN-ID value should be included in the header or if the message should be discarded. Please note that CT4 has changed the name of “3gpp-Sbi-Asserted-Plmn-Id” to “3gpp-Sbi-Originating-Network-Id” in C4-224443 since this CR was discussed last time in SA3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Roaming scenarios where one SEPP serves a given PLMN, and such PLMN has multiple PLMN-IDs but uses the same N32 connection for all PLMN-IDs, will not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.3.2, 5.9.3.2x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S3-223682, Revision of Draft CR (living document) S3-221998, revision of S3-221213, revision of S3-220951, re-submission of S3-223953 (with same proposed changes) |  |  |  |  |  |  |  |  |

*** Start Change 1 ***

#### 5.9.3.2	Requirements for Security Edge Protection Proxy (SEPP)

The SEPP shall act as a non-transparent proxy node.

The SEPP shall protect application layer control plane messages between two NFs belonging to different PLMNs or SNPNs that use the N32 interface to communicate with each other.

The SEPP shall perform mutual authentication and negotiation of cipher suites with the SEPP in the roaming network.

The SEPP shall handle key management aspects that involve setting up the required cryptographic keys needed for securing messages on the N32 interface between two SEPPs.

The SEPP shall perform topology hiding by limiting the internal topology information visible to external parties.

As a reverse proxy the SEPP shall provide a single point of access and control to internal NFs.

The receiving SEPP shall be able to verify whether the sending SEPP is authorized to use the PLMN ID or SNPN ID in the received N32 message.

The SEPP shall be able to clearly differentiate between certificates used for authentication of peer SEPPs and certificates used for authentication of intermediates performing message modifications.

NOTE 1: Such a differentiation could be done e.g. by implementing separate certificate storages.

The SEPP shall discard malformed N32 signaling messages.

The sending SEPP shall reject messages received from the NF (directly or via SCP) with JSON including "encBlockIndex" (regardless of the encoding used for that JSON request).

The receiving SEPP shall reject any message in which an IPX has inserted or relocated references to encBlockIndex.

The SEPP shall implement rate-limiting functionalities to defend itself and subsequent NFs against excessive CP signaling. This includes SEPP-to-SEPP signaling messages.

The SEPP shall implement anti-spoofing mechanisms that enable cross-layer validation of source and destination address and identifiers (e.g. FQDNs or PLMN IDs).

NOTE 2: An example for such an anti-spoofing mechanism is the following: If there is a mismatch between different layers of the message or the destination address does not belong to the SEPP’s own PLMN, the message is discarded.

The SEPP shall be able to use one or more PLMN IDs. In the situation that a PLMN is using more than one PLMN ID, this PLMN's SEPP may use the same N32-connection for all of the PLMN's PLMN IDs, with each of the PLMN's remote PLMN partners. If different PLMNs are represented by the PLMN IDs supported by a SEPP, the SEPP shall use separate N32-connections for each pair of home and visited PLMN.

In case the sending NF has not included the source PLMN ID (i.e., 3gpp-Sbi-Originating-Network-Id header) in the service/subscription request or notification message, the sending SEPP shall include the source PLMN ID (i.e., 3gpp-Sbi-Originating-Network-Id header) and send the updated signaling message to the receiving SEPP. If the sending SEPP only represents one PLMN-ID, the sending SEPP shall insert the 3gpp-Sbi-Originating-Network-Id header with its PLMN ID. In case the NF has not included the 3gpp-Sbi-Originating-Network-Id in the signaling message, and the sending SEPP represents multiple PLMN-IDs, it is up to configuration and deployment to determine which PLMN-ID value should be included in the header.

In case the NF has inserted the 3gpp-Sbi-Originating-Network-Id header in the signaling message, the sending SEPP shall compare the PLMN ID in the 3gpp-Sbi-Originating-Network-Id header in the received signaling message with the PLMN ID(s) that the sending SEPP represents as in the sending SEPP’s interconnect certificate. If the PLMN ID in the 3gpp-Sbi-Originating-Network-Id header matches with any of the PLMN IDs that the sending SEPP represents, the header is successfully verified and the sending SEPP shall forward the received signaling message to the receiving SEPP, otherwise the sending SEPP shall discard the received signaling message.

NOTE Y: The sending SEPP is not required to check that the NF is authorized since the NF and SEPP belongs to the same trust domain.

The receiving SEPP shall check if the source PLMN ID (i.e., 3gpp-Sbi-Originating-Network-Id header) included in the service/subcription request and notification messages belongs to the sending SEPP’s own PLMN. If the source PLMN ID (i.e., 3gpp-Sbi-Originating-Network-Id header) does not belong to the sending SEPP's PLMN, it is up to configuration and deployment whether the receiving SEPP discards the message or applies other measures such as raising an alarm.

NOTE X: 	For example, the receiving SEPP compares the PLMN ID in the 3gpp-Sbi-Originating-Network-Id header in the received signaling message with the PLMN-ID(s) that the sending SEPP represents as in the sending SEPP’s certificate. Alternatively, the receiving SEPP can also compare the PLMN-ID in the 3gpp-Sbi-Originating-Network-Id header with a locally configured list of PLMN-IDs that the sending SEPP represents. If the PLMN-ID in the 3gpp-Sbi-Originating-Network-Id header matches with any of the PLMN-IDs belonging to the peer sending SEPP, the header is successfully verified and the receiving SEPP forwards the received signaling message to the target NF, otherwise the receving SEPP discards the received signaling message.

The receiving SEPP shall verify that the PLMN-ID in the message itself (in case it is explicitly or implicitly contained) matches the asserted PLMN-ID in the 3gpp-Sbi-Originating-Network-Id header.

NOTE Z: The sending SEPP is not required to verify the PLMN-ID in the message itself, since it belongs to the SEPP's own PLMN and the risk mitigation by performing any such checks would be very small.

*** End Change 1 ***

*** Start Change 2 ***

#### 5.9.3.2x	Requirements for Network Functions (NF)

The NF that sends a service/subscription request or notification message shall include its PLMN-ID in the 3gpp-Sbi-Originating-Network-Id header.

In case an NF supports multiple PLMN-IDs, the sending NF shall include the PLMN ID on behalf of which the message is sent.

The handling of the PLMN-ID in the 3gpp-Sbi-Originating-Network-Id header at the receiving NF is up to configuration and deployment.

NOTE:	If the receiving NF discards the signalling message when the PLMN-ID in 3gpp-Sbi-Originating-Network-Id header does not match the expected PLMN-ID, a misconfigured PLMN-ID at the sending NF or sending SEPP can lead to service interruption.

*** End Change 2 ***