3GPP TSG-SA3 Meeting #110 	S3-231152

# Athens, Greece, 20 - 24 February 2023											(revision of S3-yyxxxx)

# Source:	Ericsson

# Title:	Updates to conclusions to KI#3 "Security for AI/ML model storage and sharing"

# Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

Approve the pCR to TR 33.738 [1] below.

# 2	References

[1]	3GPP TR 33.738 "Study on security aspects of enablers for Network Automation for 5G - phase 3"

# 3	Rationale

This contribution proposes a resolution of the Editor's Note in the conclusions to KI#3 "Security for AI/ML model storage and sharing".

# 4	Detailed proposal

*** BEGIN CHANGES ***

## 7.3	Conclusion on Key Issue #3 "Security for AI/ML model storage and sharing"

The conclusions for KI#3 are:

-	Authorization of the model retrieval at the NRF uses OAuth 2.0 token-based authorization. The NRF uses information provided by the MTLF. The NF Service Consumer registers at the NRF with its vendor ID.

-	AI/ML models can be stored in encrypted format, for end-to-end protection between MTLF and NF Service Consumer.

-	As per the request of Analytics Id by the NFc, the MTLF performs authorization of the corresponding model retrieval per selected model.

NOTE: The model delivery procedure is to be defined by 3GPP SA2.

-	ADRF verifies that the requested AI/ML model can be retrieved by the NF consumer(s) (MTLF or AnLF), based on the decision by the MTLF.

*** END CHANGES ***