3GPP TSG-SA3 Meeting #110	S3-231154

# Athens, 20 - 24 February 2023

# Source:	Ericsson

# Title:	Updates to Solution #14

# Document for:	Approval

Agenda Item:	5.16

# 1	Decision/action requested

It is proposed to add the proposed updates into the TR 33.858 [1].

# 2	References

[1]	3GPP TR 33.858 Study on security aspects of enhanced support of Non-Public Networks phase 2

[2]	3GPP TS 23.501 System Architecture for the 5G System

[3]	3GPP TS 33.501 Security architecture and procedures for 5G system

# 3	Rationale

This document proposes an update to Solution #14 “NSWO support in SNPN using any key-generating EAP-method for SNPN using CH AUSF/UDM”.

Solution #14 contains the following Editor’s notes:

Editor's Note: Whether NSWOF should be placed in CH or in SNPN is FFS and should be aligned with decisions in other working groups.

Editor's Note: Further evaluation related to the location of the NSWOF is FFS.

The first EN relates to discussions in SA2 where the architecture for NSWO is specified. However, SA2 has not yet specified the architecture for NSWO in SNPN with CH. If this use case is to be included in Rel-18, SA2 and SA3 needs to make these decisions in parallel.

The proposed architecture in Solution #14 is displayed below. It is based on the similar architecture for the roaming architecture specified in clause 4.2.15 of TS 23.501 [2].

An optional approach would be to place the NSWOF in the SNPN, with the argument that it is the SNPN providing WLAN access that also wants to provide the offloading and that the CH can then be access agnostic.

On the other hand, a CH having AUSF/UDM is one of the below:

- PLMN or another SNPN, in which case the subscription of the UE belongs to the CH. Therefore, the use case is very similar to the roaming case specified in Annex S.4 of TS 33.501 [3], where NSWOF is located in the home network.

- CH is deployed by a device manufacturer or similar e.g. when using default credentials for onboarding. In this case, WLAN offload seems highly unlikely to be used.

Observation 1 The case of NSWO in SNPN having CH AUSF/UDM is very similar to the roaming case for NSWO specified in clause 4.2.15 of TS 23.501 [2].

Observation 2: SA2 has not decided on the architecture for NSWO in SNPN having CH AUSF/UDM. But if the use case is to be included in Rel-18, SA2 and S3 need to work in parallel.

Proposal 1: It is proposed to remove the EN regarding placement of NSWOF in SNPN and align with the roaming architecture. If SA2 decides on a different architecture later, SA3 will need to adapt to it anyway.

Observation 3: If the first EN is removed there seem to be no need for further evaluation.

Proposal 2: It is proposed to remove the second EN about further evaluation being needed.

# 4	Detailed proposal

***BEGIN CHANGES***

## 6.14	Solution #14: NSWO support in SNPN using any key-generating EAP-method for SNPN using CH AUSF/UDM

### 6.14.1	Introduction

This solution solves Key issue #1 in the case of NSWO using any key-generating EAP-method in SNPN with CH AUSF/UDM.

The proposed procedure is based on the current procedures for roaming in Annex S.4 of TS 33.501[4].

### 6.14.2	Solution details

The description of NSWO for roaming defined in Annex S.4 of TS 33.501 [4] can be applied with the following adjustments:

- 	The procedures of Annex S.4 point to the roaming architecture options described in clause 4.2.15 in TS 23.501 [6].

- 	This roaming architecture is applicable by letting the Credential Holder taking the part of HPLMN. Figure 6.14.2.1 shows a modified version of Figure 4.2.15-3 of TS 23.501 [6], where HPLMN is replaced with CH and VPLMN is replaced by SNPN.

- 	The only impact on the SNPN is that it needs to deploy a AAA Proxy.

- 	The CH needs to deploy a NSWOF

- 	To be able to support all key-generating EAP-mechanisms, the procedures of Solution #9 of this document can be applied on the proposed architecture.

Figure 6.14.2.1 Proposed roaming reference architectures to support authentication for Non-seamless WLAN offload in SNPN with CH AUSF/UDM

### 6.14.3	System impact

The solution has impact on UE (needs to support NSWO).SNPN needs to deploy AAA proxy and CH needs to deploy NSWOF.

### 6.14.4	Evaluation

This solution solves Key issue #1 in aspect of supporting NSWO in SNPN that deploys CH AUSF/UDM. It reuses the procedures of Annex S of TS 33.501 [4] as much as possible adding the possibility of using any key-generating EAP-method.

***END CHANGES***