3GPP TSG-SA3 Meeting #110 	S3-231179

# Athens, Greece meeting, 20 - 24 January 2023

# Source:	Philips International B.V.

# Title:	Evaluation Solution #10

# Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This Tdoc proposes further evaluation of Sol#10 in TR 33.740. We kindly request SA3 to review and approve this contribution.

# 2	References

[1]	Hao, Feng, and Paul C. van Oorschot. "SoK: Password-Authenticated Key Exchange--Theory, Practice, Standardization and Real-World Lessons." Proceedings of the 2022 ACM on Asia Conference on Computer and Communications Security. 2022.

[2]	Joern-Marc Schmidt, RFC 8125: “Requirements for Password-Authenticated Key Agreement (PAKE) Schemes”, April 2017, available online at https://datatracker.ietf.org/doc/rfc8125/

# 3	Rationale

This Tdoc provides further evaluation of Sol. #10. We kindly request SA3 to review and approve this contribution.

The definition of long-term credentials in TS 33.536 states the following:

“Long term credentials: These are the credentials that are provisioned into the UE(s) and form the root of the security of the PC5 unicast link. The credentials may include symmetric key(s) or public/private key pair depending on the particular use case.”

The definition in TS 33.536 enables multiple protocols for key establishment and authentication based on symmetric keys or public key cryptography; but it leaves out PAKEs.

However, PAKE-based schemes are very useful schemes that have been adopted in real-world commercial systems for various use cases such as credential recovery (e.g., SRP-6a in iCloud), device pairing (e.g., third-generation e-passports and readers mutual authentication with Machine Readable Zone (MRZ) data used as shared secret), or end-to-end security (e.g., J-PAKE in Thread) [1]. IRTF has also performed extensive research and standardisation in this area [2].

PAKEs are a type of protocol for authentication and key establishment that are based on passwords. The passwords as such are a type of credentials that may valid for a short or for a long period and may be bound to different access rights. There are multiple advantages when using PAKEs:

- Passwords may be short, and still, PAKEs allow for strong confidentiality;

- Authentication is based on the passwords so that it allows for authentication/authorization without the need of a public key infrastructure;

- passwords may be short, thus, passwords may be easily entered in a device by a user, e.g., to provide authentication in out-of-coverage scenarios or in emergency situations;

- In augmented PAKEs, the password is not stored in the clear providing higher security;

- ...

The passwords used in a PAKE may also be long-term credentials. Thus, PAKEs do not seem to introduce any drawbacks in comparison with other schemes relying on other types of long-term credentials.

Thus, it is proposed to remove the Editor’s Note and add the following text:

“TS 33.536 defines long-term credentials as the credentials that are provisioned into the UE(s) and form the root of the security of the PC5 unicast link. The credentials may include symmetric key(s) or public/private key pair depending on the particular use case. The definition in TS 33.536 enables multiple protocols for key establishment and authentication based on symmetric keys or public key cryptography; but it leaves out PAKEs. However, PAKE-based schemes are very useful schemes that have been adopted in real-world commercial systems for various use cases such as credential recovery (e.g., SRP-6a in iCloud), device pairing (e.g., third-generation e-passports and readers mutual authentication with Machine Readable Zone (MRZ) data used as shared secret), or end-to-end security (e.g., J-PAKE in Thread) [1]. IRTF has also performed extensive research and standardisation in this area [2].	PAKEs are a type of protocol for authentication and key establishment that are based on passwords. The passwords as such are a type of credentials that may valid for a short or for a long period and may be bound to different access rights. There are multiple advantages when using PAKEs:

- Passwords may be short, and still, PAKEs allow for strong confidentiality;

- Authentication is based on the passwords so that it allows for authentication/authorization without the need of a public key infrastructure;

- passwords can be bound to access rights, and thus, provide a convenient approach for authorization;

- passwords may be short, thus, passwords may be easily entered in a device by a user, e.g., to provide authentication in out-of-coverage scenarios or in emergency situations;

- In augmented PAKEs, the password is not stored in the clear providing higher security;

The passwords used in a PAKE may also be long-term credentials. Thus, PAKEs do not seem to introduce any drawbacks in comparison with other schemes relying on other types of long-term credentials.”

# 4	Detailed proposal

*** Start changes ***

# 2         References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non‑specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-33: "Study on System enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 2".

[3]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[4]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[5]	3GPP TS 22.115: " Service aspects; Charging and billing".

[6]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[7]	3GPP TR 33.870: "Study of privacy of identifiers over radio access".

[8]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[9]	3GPP TS 33.536: "Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

[10]	IRTF “CPace, a balanced composable PAKE” available online at https://datatracker.ietf.org/doc/draft-irtf-cfrg-cpace/

[11]	IRTF “The OPAQUE Asymmetric PAKE Protocol” available online at https://datatracker.ietf.org/doc/draft-irtf-cfrg-opaque/

[12]	IRTF “SPAKE2, a PAKE” available online at https://datatracker.ietf.org/doc/draft-irtf-cfrg-spake2/

[13]	IRTF “SPAKE2+, an Augmented PAKE” available online at https://datatracker.ietf.org/doc/draft-bar-cfrg-spake2plus/

[14]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[15]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[16]	3GPP TS 22.101: "Service aspects; Service principles".

[x]	Hao, Feng, and Paul C. van Oorschot. "SoK: Password-Authenticated Key Exchange--Theory, Practice, Standardization and Real-World Lessons." Proceedings of the 2022 ACM on Asia Conference on Computer and Communications Security. 2022.

[y]	Joern-Marc Schmidt, RFC 8125: “Requirements for Password-Authenticated Key Agreement (PAKE) Schemes”, April 2017, available online at https://datatracker.ietf.org/doc/rfc8125/

*** Next changes ***

### 6.10.3	Evaluation

This solution addresses Key Issue #2 and Key Issue #3 by describing how ProSe can support the usage of PAKEs. This allows setting up a secure/authenticated link and authorizing it based on the usage of passwords.

Impact on ProSe is limited to the exchange of information/messages required to support a PAKE:

- The configuration of Source-UE, Target-UE, UE-to-UE relay with PAKE parameters as in Clause 6.10.2.1.

- The derivation of the PAKE parameters and password from the “raw-password” and the associated metadata fields as in Clause 6.10.2.1.

- The exchange of PAKE related parameters required for the PAKE execution as in Clause 6.10.2.3.

- The usage of PAKE derived keys as in Clause 6.10.2.5.

- The configuration of a policy determining whether a user is allowed to enter a password when Source-UE/Target-UE/UE-to-UE relay is/are out-of-coverage.

This solution allows authorizing the Source-UE, Target-UE, UE-to-UE relay based on the configured password information. This provides a simple approach to ensure that the involved UEs are authenticated and authorized to communicate with each other regardless of their coverage situation.

TS 33.536 defines long-term credentials as “the credentials that are provisioned into the UE(s) and form the root of the security of the PC5 unicast link. The credentials may include symmetric key(s) or public/private key pair depending on the particular use case”. This definition in TS 33.536 enables multiple protocols for key establishment and authentication based on symmetric keys or public key cryptography; but it leaves out PAKEs. However, PAKE-based schemes are very useful and have been adopted in real-world commercial systems for various use cases such as credential recovery (e.g., SRP-6a in iCloud), device pairing (e.g., third-generation e-passports and readers’ mutual authentication with Machine Readable Zone (MRZ) data used as a shared secret), or end-to-end security (e.g., J-PAKE in Thread) [x]. IRTF has also performed extensive research and standardisation in this area [y]. PAKEs are a type of protocol for authentication and key establishment that are based on passwords. Passwords as such are a type of credentials that may be valid for a variable (e.g., short or long) time period. There are multiple advantages when using PAKEs:

- passwords may be short, and still, PAKEs allow for strong confidentiality;

- authentication is based on passwords hence allowing for authentication/authorization without the need for a public key infrastructure;

- passwords can be bound to access rights, and thus, provide a convenient approach for authorization;

- passwords may be short, thus, passwords may be easily entered in a device by a user, e.g., to provide authentication in out-of-coverage scenarios or in emergency situations when no other credentials are available;

- in augmented PAKEs, the password is not stored in the clear, thus providing higher security and protection against impersonation.

The passwords used in a PAKE may also be “long-term credentials”. Thus, PAKEs do not seem to introduce any drawbacks in comparison with other schemes relying on other types of long-term credentials.

*** End changes ***