3GPP TSG-SA3 Meeting #110 	S3-231191

# Athens, Greece, 20 - 24 February 2023											(revision of S3-yyxxxx)

# Source:	Nokia, Nokia Shanghai Bell

# Title:	pCR against Living CR - SEPP to include and verify the source PLMN ID

# Document for:	Approval, Information, Discussion

Agenda Item:	4.7

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	S3-223953 Living CR

# 3	Rationale

In a roaming scenario where one SEPP serves a given PLMN, and such PLMN has multiple PLMN-IDs but uses the same N32 connection for all PLMN-IDs, the current specification does not describe how to identify and verify the source PLMN-ID of a message.

This pCR is against S3-223953 (Living CR).

# 4	Detailed proposal

*** Start Change 1 ***

#### 5.9.3.2	Requirements for Security Edge Protection Proxy (SEPP)

The SEPP shall act as a non-transparent proxy node.

The SEPP shall protect application layer control plane messages between two NFs belonging to different PLMNs or SNPNs that use the N32 interface to communicate with each other.

The SEPP shall perform mutual authentication and negotiation of cipher suites with the SEPP in the roaming network.

The SEPP shall handle key management aspects that involve setting up the required cryptographic keys needed for securing messages on the N32 interface between two SEPPs.

The SEPP shall perform topology hiding by limiting the internal topology information visible to external parties.

As a reverse proxy the SEPP shall provide a single point of access and control to internal NFs.

The receiving SEPP shall be able to verify whether the sending SEPP is authorized to use the PLMN ID or SNPN ID in the received N32 message.

The SEPP shall be able to clearly differentiate between certificates used for authentication of peer SEPPs and certificates used for authentication of intermediates performing message modifications.

NOTE 1: Such a differentiation could be done e.g. by implementing separate certificate storages.

The SEPP shall discard malformed N32 signaling messages.

The sending SEPP shall reject messages received from the NF (directly or via SCP) with JSON including "encBlockIndex" (regardless of the encoding used for that JSON request).

The receiving SEPP shall reject any message in which an IPX has inserted or relocated references to encBlockIndex.

The SEPP shall implement rate-limiting functionalities to defend itself and subsequent NFs against excessive CP signaling. This includes SEPP-to-SEPP signaling messages.

The SEPP shall implement anti-spoofing mechanisms that enable cross-layer validation of source and destination address and identifiers (e.g. FQDNs or PLMN IDs).

NOTE 2: An example for such an anti-spoofing mechanism is the following: If there is a mismatch between different layers of the message or the destination address does not belong to the SEPP’s own PLMN, the message is discarded.

The SEPP shall be able to use one or more PLMN IDs. In the situation that a PLMN is using more than one PLMN ID, this PLMN's SEPP may use the same N32-connection for all of the PLMN's PLMN IDs, with each of the PLMN's remote PLMN partners.

If different PLMNs are represented by the PLMN IDs supported by a SEPP, the SEPP shall use separate N32-connections for each pair of home and visited PLMN. The SEPP shall have a certificate for each of the PLMNs it serves.

<<<< START OF TEXT IN LIVING CR >>>

<<<< ALL NEW TEXT -  revisions shown against Living CR>>>>

A sending NF complying with this release shall include the source PLMN ID (i.e., 3gpp-Sbi-Originating-Network-Id header) in the signalling message (service/subscription request or notification message).

For legacy NFs and SCPs, pre-release 17, the following applies with respect to the source PLMN ID (i.e., 3gpp-Sbi-Originating-Network-Id header) handling:

- 	In case the sending NF has not included the 3gpp-Sbi-Originating-Network-Id header in the signalling message and the message is sent via SCP to the sending SEPP, the sending SCP should include the 3gpp-Sbi-Originating-Network-Id header if it is able to determine it.

-	In case the sending NF and the SCP have not included the 3gpp-Sbi-Originating-Network-Id header in the signalling message , the sending SEPP shall include the 3gpp-Sbi-Originating-Network-Id header and send the updated signaling message to the receiving SEPP.

-	If the sending SEPP only represents one PLMN-ID, the sending SEPP shall insert the 3gpp-Sbi-Originating-Network-Id header with this ID.

-	If the sending SEPP represents multiple PLMN-IDs, it is up to configuration and deployment to determine which PLMN-ID value should be included in the header.

Sending SEPP behavior:

- 	In case the sending NF or the SCP has inserted the 3gpp-Sbi-Originating-Network-Id header in the signaling message, the sending SEPP shall compare the PLMN ID in the 3gpp-Sbi-Originating-Network-Id header in the received signaling message with the PLMN ID(s) that the sending SEPP represents by its certificate.

- 	If the PLMN ID does not match with any of the PLMN IDs (or SNPN IDs) that the sending SEPP represents, the sending SEPP shall discard the received signaling message.

-	If the PLMN ID matches with any of the PLMN IDs that the sending SEPP represents, the sending SEPP shall forward the signaling message to the receiving SEPP.

Receiving SEPP behavior:

- 	The receiving SEPP shall check if the 3gpp-Sbi-Originating-Network-Id header)included in the signalling message belongs to the sending SEPP’s own PLMN. It does this by verifying that the asserted PLMN ID in the 3gpp-Sbi-Originating-Network-Id header matches the sending SEPP's own PLMN.

- If the 3gpp-Sbi-Originating-Network-Id header does not match with any of the PLMN IDs belonging to the peer sending SEPP, the receving SEPP discards the received signaling message.

- If the 3gpp-Sbi-Originating-Network-Id header matches with any of the PLMN IDs IDs belonging to the peer sending SEPP, the header is successfully verified, and the receiving SEPP forwards the received signaling message to the target NF.

*** End Change 1 ***

*** Start Change 2 ***

>>> ALL NEW TEXT. revisions shown against Living CR >>>>

#### 5.9.3.2x	Requirements for Network Functions (NF)

The NF that sends a service/subscription request or notification message shall include its PLMN-ID in the 3gpp-Sbi-Originating-Network-Id header.

In case an NF supports multiple PLMN-IDs, the sending NF shall include the PLMN ID in the 3gpp-Sbi-Originating-Network-Id header on behalf of which the message is sent.

The handling of the PLMN-ID in the 3gpp-Sbi-Originating-Network-Id header at the receiving NF is up to configuration and deployment.

NOTE:	A misconfigured PLMN ID in the 3gpp-Sbi-Originating-Network-Id header can lead to service interruption.

*** End Change 2 ***

FOR REFERENCE – below are the revisions against 33.501 v17.8.0

******* START OF CHANGES

#### 5.9.3.2	Requirements for Security Edge Protection Proxy (SEPP)

The SEPP shall act as a non-transparent proxy node.

The SEPP shall protect application layer control plane messages between two NFs belonging to different PLMNs or SNPNs that use the N32 interface to communicate with each other.

The SEPP shall perform mutual authentication and negotiation of cipher suites with the SEPP in the roaming network.

The SEPP shall handle key management aspects that involve setting up the required cryptographic keys needed for securing messages on the N32 interface between two SEPPs.

The SEPP shall perform topology hiding by limiting the internal topology information visible to external parties.

As a reverse proxy the SEPP shall provide a single point of access and control to internal NFs.

The receiving SEPP shall be able to verify whether the sending SEPP is authorized to use the PLMN ID or SNPN ID in the received N32 message.

The SEPP shall be able to clearly differentiate between certificates used for authentication of peer SEPPs and certificates used for authentication of intermediates performing message modifications.

NOTE 1: Such a differentiation could be done e.g. by implementing separate certificate storages.

The SEPP shall discard malformed N32 signaling messages.

The sending SEPP shall reject messages received from the NF (directly or via SCP) with JSON including "encBlockIndex" (regardless of the encoding used for that JSON request).

The receiving SEPP shall reject any message in which an IPX has inserted or relocated references to encBlockIndex.

The SEPP shall implement rate-limiting functionalities to defend itself and subsequent NFs against excessive CP signaling. This includes SEPP-to-SEPP signaling messages.

The SEPP shall implement anti-spoofing mechanisms that enable cross-layer validation of source and destination address and identifiers (e.g. FQDNs or PLMN IDs).

NOTE 2: An example for such an anti-spoofing mechanism is the following: If there is a mismatch between different layers of the message or the destination address does not belong to the SEPP’s own PLMN, the message is discarded.

The SEPP shall be able to use one or more PLMN IDs. In the situation that a PLMN is using more than one PLMN ID, this PLMN's SEPP may use the same N32-connection for all of the PLMN's PLMN IDs, with each of the PLMN's remote PLMN partners.

If different PLMNs are represented by the PLMN IDs supported by a SEPP, the SEPP shall use separate N32-connections for each pair of home and visited PLMN. The SEPP shall have a certificate for each of the PLMNs it serves.

A sending NF complying with this release shall include the source PLMN ID (i.e., 3gpp-Sbi-Originating-Network-Id header) in the signalling message (service/subscription request or notification message).

For legacy NFs and SCPs, pre-release 17, the following applies with respect to the source PLMN ID (i.e., 3gpp-Sbi-Originating-Network-Id header) handling:

- 	In case the sending NF has not included the 3gpp-Sbi-Originating-Network-Id header in the signalling message and the message is sent via SCP to the sending SEPP, the sending SCP should include the 3gpp-Sbi-Originating-Network-Id header if it is able to determine it.

-	In case the sending NF and the SCP have not included the 3gpp-Sbi-Originating-Network-Id header in the signalling message, the sending SEPP shall include the 3gpp-Sbi-Originating-Network-Id header and send the updated signaling message to the receiving SEPP.

-	If the sending SEPP only represents one PLMN-ID, the sending SEPP shall insert the 3gpp-Sbi-Originating-Network-Id header with this ID.

-	If the sending SEPP represents multiple PLMN-IDs, it is up to configuration and deployment to determine which PLMN-ID value should be included in the header.

Sending SEPP behavior:

- 	In case the sending NF or the SCP has inserted the 3gpp-Sbi-Originating-Network-Id header in the signaling message, the sending SEPP shall compare the PLMN ID in the 3gpp-Sbi-Originating-Network-Id header in the received signaling message with the PLMN ID(s) that the sending SEPP represents by its certificate.

- 	If the PLMN ID does not match with any of the PLMN IDs (or SNPN IDs) that the sending SEPP represents, the sending SEPP shall discard the received signaling message.

-	If the PLMN ID matches with any of the PLMN IDs that the sending SEPP represents, the sending SEPP shall forward the signaling message to the receiving SEPP.

Receiving SEPP behavior:

- 	The receiving SEPP shall check if the 3gpp-Sbi-Originating-Network-Id header) included in the signalling message belongs to the sending SEPP’s own PLMN. It does this by verifying that the asserted PLMN ID in the 3gpp-Sbi-Originating-Network-Id header matches the sending SEPP's own PLMN.

- If the 3gpp-Sbi-Originating-Network-Id header does not match with any of the PLMN IDs belonging to the peer sending SEPP, the receving SEPP discards the received signaling message.

- If the 3gpp-Sbi-Originating-Network-Id header matches with any of the PLMN IDs IDs belonging to the peer sending SEPP, the header is successfully verified, and the receiving SEPP forwards the received signaling message to the target NF.

******* NEXT CHANGE

#### 5.9.3.2x	Requirements for Network Functions (NF)

The NF that sends a service/subscription request or notification message shall include its PLMN-ID in the 3gpp-Sbi-Originating-Network-Id header.

In case an NF supports multiple PLMN-IDs, the sending NF shall include the PLMN ID in the 3gpp-Sbi-Originating-Network-Id header on behalf of which the message is sent.

The handling of the PLMN-ID in the 3gpp-Sbi-Originating-Network-Id header at the receiving NF is up to configuration and deployment.

NOTE:	A misconfigured PLMN ID in the 3gpp-Sbi-Originating-Network-Id header can lead to service interruption.

******* END OF CHANGES