3GPP TSG-SA3 Meeting #110 	S3-231291

# Athens, Greece, 20 - 24 February 2023											(revision of S3-yyxxxx)

# Source:	Ericsson

# Title:	Conclusion for key issue #1

# Document for:	Approval

Agenda Item:	5.15

# 1	Decision/action requested

This contribution provides a conclusion for key issue #1.

# 2	References

[1]	3GPP TR 33.890 v0.5.0: Study on security support for Next Generation Real Time Communication services

# 3	Rationale

This contribution provides a conclusion for key issue #1.

# 4	Detailed proposal

It is proposed that the following pCR is included in the TR [1].

**** START OF CHANGES ****

## 7.X		Conclusion for Key Issue #1

For KI#1,it is proposed to use the following guidelines based on Solution#1 and Solution#2:

1)	The IMPU of the calling UE shall be included in the SIP INVITE received by the originating IMS. The 3rd party ID of the calling UE may or may not be included in the SIP INVITE received by the originating IMS.

2)	The originating IMS checks whether the IMS subscription of the calling PBX or UE is authorized to use 3rd party ID. If the calling PBX or UE is authorized to use 3P IDs, the originating IMS subsystem gets Rich Call Data of 3rd party subscriber from a Database based on the received IMPU. Originating IMS network is assumed to have a secure channel to the Database which includes Rich Call Data information. The address of the applicable Database for the user can be stored in HSS.

NOTE:	If no third-party ID info is received in the SIP INVITE from the UE or if there is a mismatch between the received 3rd party ID in the SIP INVITE and data retrieved from the Database based on the IMPU, it is left for originating IMS network’s implementation.

3)	Signing and verification of the SIP Identity header which includes the telephone number and Rich Call Data are based on STIR/SHAKEN Reference Architecture and TS 24.229 [4].

**** END OF CHANGES ****