3GPP TSG-SA3 Meeting #110 	S3-231292

# Athens, Greece, 20 - 24 February 2023											(revision of S3-yyxxxx)

# Source:	Ericsson

# Title:	Conclusion for key issue #2

# Document for:	Approval

Agenda Item:	5.15

# 1	Decision/action requested

This contribution provides a conclusion for key issue #2.

# 2	References

[1]	3GPP TR 33.890 v0.5.0: Study on security support for Next Generation Real Time Communication services

# 3	Rationale

This contribution provides a conclusion for key issue #2.

# 4	Detailed proposal

It is proposed that the following pCR is included in the TR [1].

**** START OF CHANGES ****

## 7.X	Conclusion for Key Issue #2

The following conclusions are agreed for KI#2:

A)	If the external Data Server supports only HTTP, the DCMF/enhanced MRF will terminate DTLS for the following cases:

1)	Bootstrap data channel;

2)	P2A/A2P application data channel;

3)	P2P application data channel with the involvement of DCMF/enhanced MRF;

4)	For P2P application data channel without the involvement of DCMF/enhanced MRF, DTLS is terminated in the eIMS-AGW.

B)	If the external Data Server supports both HTTP and IETF RFC 8831 WebRTC data channel protocols, the end-to-end (e2e) encryption for the data channel should be used as specified in IETF RFC 8831 [9] and IETF RFC 8827 [13] for the P2A/A2P Application data channel use cases.

Editor's Note: LI considerations are FFS.

**** END OF CHANGES ****