3GPP TSG-SA3 Meeting #110	S3-231378

# Athens (Greece), 20 - 24 February 2023                                       revision of S3-231376

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.203 | CR | 0268 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security vulnerability fix for use of AES-GCM and AES-GMAC in 33.203 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCryptPr |  |  |  |  |  | Date: |  |  | 2023-2-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Background: When AES-GCM or AES-GMAC are in used for IPsec, it is vital that the nonce supplied to the algorithm for encrypting each packet is never repeated under the same key. To achieve this, the encrypter for an IPsec Security Association (SA) is required to generate an Initialization Vector (IV) for each encrypted packet that is not repeated for the lifetime of the SA. IPsec does not provide a method for peers to coordinate IV generation across SAs, so it is possible for IVs to be duplicated across multiple SAs. In order to ensure this does not lead to nonce reuse under the same key, a salt value is supplied to IPsec along with the key, which is combined with each IV to form the nonce supplied to the algorithm. If two or more IPsec SAs use the same key, it is required that the salt values supplied for the SAs be different. According to Section 10 of RFC 4106, which describes the use of AES-GCM in IPsec ESP: “When IKE is used to establish fresh keys between two peer entities, separate keys are established for the two traffic flows. If a different mechanism is used to establish fresh keys (one that establishes only a single key to encrypt packets), then there is a high probability that the peers will select the same IV values for some packets. Thus, to avoid counter block collisions, ESP implementations that permit use of the same key for encrypting and decrypting packets with the same peer MUST ensure that the two peers assign different salt values to the security association (SA)” Security threats: The current version of Annex I specifies that all 4 IPsec SAs established for SIP security will use the same salt value, which violates the above security requirement from IETF and thus can lead to nonce resue under the same key. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document proposes a modification to the salt generation procdeure to provide unique salt values for each SA by XOR’ing the value output from the KDF with the SPI for the SA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Vulnerable Implementations violating IETF security requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex I |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGES ***

Annex I (normative):
Key expansion functions for IPsec ESP

Integrity Keys:

If the selected authentication algorithm is HMAC-SHA-1-96 then IKESP is obtained from IKIM by appending 32 zero bits to the end of IKIM to create a 160-bit string.

If selected authentication algorithm is AES-GMAC as specified in RFC 4543 [74] with 128 bit key then IKESP = IKIM.

The salt value specified in Section 3.2 of RFC 4543 [74] shall be derived using the key derivation function KDF defined in Annex B of TS 33.220 [66]. The input Key to the KDF function shall be equal to the concatenation of CKIM and IKIM: CKIM || IKIM. The input S to the KDF function shall be formed from the following parameters:

-	FC = 0x58.

-	P0 = "AES_GMAC_SALT" .

-	L0 = length of the string “AES_GMAC_SALT” (i.e. 0x00 0x0D).

The salt value for each IPsec SA shall consist of the 32 least significant bits of the 256 bits of the KDF output XOR’d with the SA’s SPI (represented as a 32-bit integer in network byte order).

"Hmac-sha-1-96" is not recommended.

Encryption Keys:

If selected encryption algorithm is AES-CBC as specified in RFC 3602 [22] with 128 bit key then CKESP = CKIM .

If selected encryption algorithm is AES-GCM as specified in RFC 4106 [73] with 128 bit key then CKESP = CKIM. The salt value specified in Section 4 of RFC 4106 [73] shall be derived using the key derivation function KDF defined in Annex B of TS 33.220 [66]. The input Key to the KDF function shall be equal to the concatenation of CKIM and IKIM: CKIM || IKIM. The input S to the KDF function shall be formed from the following parameters:

-	FC = 0x59

-	P0 = “AES_GCM_SALT”

-	L0 = length of the string “AES_GCM_SALT” (i.e. 0x00 0x0C)

The salt value for each IPsec SA shall consist of the 32 least significant bits of the 256 bits of the KDF output XOR’d with the SA’s SPI (represented as a 32-bit integer in network byte order)

"aes-cbc" is not recommended.

*** END OF CHANGES ***