3GPP TSG-SA3 Meeting #110 	draft_S3-231434-r2

# Athens, Greece, 20 - 24 February 2023											(revision of S3-230982)

# Source:	Huawei, HiSilicon

# Title:	General description about secondary authentication

# Document for:	Approval

Agenda Item:	4.10

1	Decision/action requested

To approve the procedure and the procedure can be merged into the living doc.

2	References

NA

3	Rationale

This contribtion proposes Secondary Authentication Procedure for Remote UE without N3IWF based on the S3-222400.

4	Detailed proposal

It is proposed to approve the following new procedure and the content can be merged to the living doc.

*** BEGIN of 1st CHANGE ***

6.3.3.3.x.1	General

This clause specifies the 5G Prose Remote UE specific secondary authentication between a 5G ProSe Remote UE, via a 5G ProSe Layer-3 UE-to-Network Relay without N3IWF and an external Data Network (DN) based on network-controlled authorization (i.e. using 5G ProSe Remote UE specific authentication) as described in clause 6.3.3.3.2. This procedure is optional to support. This procedure is different from the secondary authentication defined in TS 33.501 [3], as the 5G Prose Remote UE may not register to the 5G ProSe UE-to-Network Relay’s network, and requests to use 5G ProSe UE-to-Network Relay’s PDU session. In this case, the 5G ProSe UE-to-Network Relay’s SMF needs to check whether the 5G ProSe UE-to-Network Relay’s PDU session can be used by the 5G Prose Remote UE by performing the secondary authentication that is defined in TS 33.501[3].

The SMF of the 5G ProSe UE-to-Network Relay triggers the secondary authentication of the 5G ProSe Remote UE based on the subscription information and the local configuration of the SMF when it receives a NAS message (e.g. Remote UE Report) from the 5G ProSe UE-to-Network Relay.

The EAP framework specified in IETF RFC 3748 [12] shall be used for authentication between the 5G ProSe Remote UE and a DN-AAA server in the external data network.

Following clause describes the procedures for initial secondary authentication of the 5G ProSe Remote UE with the external DN-AAA server.

*** END of CHANGE ***