3GPP TSG-SA3 Meeting #110 	draft S3-231475-r1

# Athens, Greece, 20 - 24 February 2023			(merger of S3-230898 and S3-2301371)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.222 | CR | draftCR | rev | <-> | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Shared key-based mutual authentication between UE and NAF to TS 33.222 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA_GBA_DTLS |  |  |  |  |  | Date: |  |  | 2023-2-6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In order to complete the work on the DTLS for GBA Ua protocol, it is required to specify the DTLS profile for shared key-based mutual authentication between UE and NAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add DTLS profile for shared key-based mutual authentication between UE and NAF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete work for the DTLS for GBA Ua protocol |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of Change 1****

Annex X (normative): 
DTLS profiles for GBA Ua

# X.1	DTLS based protocols

## X.1.1	General

This Annex covers the aspects specific to the DTLS profiles for GBA Ua.

## X.1.2	Shared key-based UE authentication with certificate-based AF authentication

## X.1.3	Shared key-based mutual authentication between UE and AF

#### X.1.3.1	General

The TLS profile specified in TS 33.210 [13] clause 6.2 apply to DTLS 1.3[x].

#### X.1.3.3	Procedures for DTLS 1.3

The procedures and PSK identities given in clause 5.4.0.2 of TS 33.222 for TLS 1.3 are also applicable for DTLS 1.3, with the following changes.

At step 1, If the DTLS client resides in the ME, Ks_(ext)_NAF shall be used as the GBA shared key. If the DTLS client resides in the UICC, Ks_int_NAF shall be used as the GBA shared key. In the selection of the GBA mode by the UE, AKA-based modes shall take priority over GBA_Digest.

The UE derives the DTLS premaster secret from the NAF specific key (Ks_(ext)_NAF if the initiating DTLS client resides on the ME or Ks_int_NAF if the initiating DTLS client resides on the UICC).

PSK identities should be delivered via DTLS message.

Editor’s Note: The steps are to be updated to remove any dependency on application layer protocol (here HTTPS).

**** End of Change 1****

**** Start of Change 2****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.002: "Network architecture".

[2]	3GPP TS 22.250: "IP Multimedia Subsystem (IMS) group management"; Stage 1".

[3]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture".

[4]	Void

[5]	3GPP TS 33.141: "Presence Service; Security".

[6]	Void.

[7]	Void.

[8]	Void.

[9]	IETF RFC 2818 (2000): "HTTP Over TLS".

[10]	Void

[11]	Void

[12]	Void

[13]	3GPP TS 33.210: "3G Security; Network Domain Security; IP network layer security".

[14]	Void.

[15]	 Void.

[16]	3GPP TS 33.221: "Generic Authentication Architecture (GAA); Support for subscriber certificates".

[17]	Void.

[18]	3GPP TS 24.109: "Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[19]	3GPP TS 29.109: "Generic Authentication Architecture (GAA), Zh and Zn Interface based on the Diameter protocol; Stage 3".

[20]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[21]	Void.

[22]	 Void.

[23]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[24]	W3C Working Draft (Jan 22, 2013): "HTML5.1 Nightly – A vocabulary and associated APIs for HTML and XHTML", work in progress, http://dev.w3.org/html5/spec/.

[25]	IETF RFC 5929 (2010): "Channel Bindings for TLS".

[26]	W3C Working Draft (Oct 20, 2011): "File API", work in progress, http://www.w3.org/TR/FileAPI/.

[27]	W3C Candidate Recommendation (Dec 8, 2011): "Web Storage", work in progress, http://www.w3.org/TR/webstorage/

[28]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[29]	IETF RFC 5705 (2010): "Keying Material Exporters for Transport Layer Security (TLS)".

[30]	IETF RFC 8446 (2018): "The Transport Layer Security (TLS) Protocol Version 1.3".

[31]	IETF RFC 7235: "Hypertext Transfer Protocol (HTTP/1.1): Authentication".

[32]	IETF RFC 7616: "HTTP Digest Access Authentication".

[33]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[x]	IETF RFC 9146: "The Datagram Transport Layer Security (DTLS) Protocol Version 1.3"

**** End of Change 1****