3GPP TSG-SA3 Meeting #110 	S3-231497

# Athens, Greece, 20 - 24 February 2023(merger of S3-230842, S3-230922, S3-230925, S3-231151)

# Source:	Ericsson, China Telecommunications, Intel, Nokia, Nokia Shanghai Bell

# Title:	Conclusions to KI#2 "Authorization of selection of participant NWDAF instances in the Federated Learning group"

# Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

Approve the pCR to TR 33.738 [1] below.

# 2	References

[1]   3GPP TR 33.738 0.5.0 "Study on security aspects of enablers for Network Automation for 5G - phase 3"

[2]   3GPP TR 23.700-81 V18.0.0 " Study of Enablers for Network Automation for 5G System (5GS); Phase 3"

# 3	Rationale

This pCR proposes to use the ideas of solution #17 for Authorization of selection of participant NWDAF instances in the Federated Learning group to conclude KI#2.

# 4	Detailed proposal

*** BEGIN OF CHANGES ***

## 7.X	Conclusion on Key Issue #2 "Authorization of selection of participant NWDAF instances in the Federated Learning group"

It is recommended to use the following general principles for authorization of participant NWDAFs in a Federated Learning group:

- Initial registration of the NWDAFs involved in FL including Analytics Id and FL capability type (i.e. client, server).

-	Authorization of the server NWDAF to include a client NWDAF into a Federated Learning group is done by NRF using SBA OAuth 2.0 token-based authorization.

-	The NRF uses the interoperability indicator, specified in TS 23.288 [5] and provided by the client NWDAF during registration, for authorization of the server NWDAF.

Editor's Note: Whether additional IEs specific for authorization of server NWDAFs need to be included in the access token and access token request is ffs. Proposed candidates are: list of permitted server NWDAF(s), analytics id, list of permitted FL server for each analytics id, FL group ID, ML model ID.

-	Authorization of the client NWDAF is implicit, since it can join a Federated Learning group only when selected by the server NWDAF.

*** END OF CHANGES ***