3GPP TSG-SA3 Meeting #110	S3-231501

# Athens, 20 - 24 February 2023	 merger of S3-231157, S3-230947

# Source:	Ericsson

# Title:	Updated conclusion for KI#1 regarding NSWO

# Document for:	Approval

Agenda Item:	5.16

# 1	Decision/action requested

It is proposed to add the proposed conclusions into the TR 33.858 [1].

# 2	References

[1]	3GPP TR 33.858 Study on security aspects of enhanced support of Non-Public Networks phase 2

# 3	Rationale

For NSWO access there are three cases:

- NSWO access to SNPN having AUSF/UDM (Solution #9)

- NSWO access to CH using AUSF/UDM (Solution #14)

- NSWO access using SNPN credentials from CH AAA (Solution #15)

This document proposes to conclude on the above cases and proposed solutions.

# 4	Detailed proposal

***BEGIN CHANGES***

## 7.1	Conclusions for KI#1 Security of non-3GPP access for SNPN

### 7.1.1	Scope

TR 23.700-08 [2] has concluded in clause 8.2 that N3GPP access to SNPN includes the following types of access:

- 	Untrusted/Trusted N3GPP access including support for onboarding

- 	NSWO access to SNPN using SNPN credentials

The case of N5CW devices has not been addressed by TR 23.700-08 [2], but there are solutions for this case proposed in this study.

### 7.1.2 Conclusion for Untrusted N3GPP access to SNPN

Solution #1 is selected as basis for normative work for untrusted access to SNPN.

This means that the procedure specified in TS 33.501 [2] clause 7.2.1 will be reused for normative work with the following modifications:

- 	Support for all key generating EAP-methods: Extend the applicable authentication mechanism in step 7 to key-generating EAP authentication methods.

- 	Support for onboarding: Add possibility to send onboarding SUCI in step 5

- 	Support for usage of anonymous SUCI: Add possibility to send anonymous SUCI in step 5 (also affecting steps 6 and 7) if the construction of SUCI as described in clause 6.12 of TS 33.501 [2] cannot be used and if the employed EAP method supports privacy.

Editor's Note: It is FFS if the EAP verification result in step 8 needs to be made mandatory.

Editor’s Note: Further conclusions are FFS.

### 7.1.3 Conclusion for Trusted N3GPP access to SNPN

Solution #2 is selected as basis for normative work with regards to the aspects:

- 	Support for all key generating EAP-methods

- 	Support for onboarding

This implies that the procedure specified in TS 33.501 [2] section 7A.2.4 will be reused for normative work with the following modifications:

- 	Support for usage of anonymous SUCI:

- 	Add possibility to send anonymous SUCI in step 5 (affecting also following steps 5-8) if the construction of SUCI as described in clause 6.12 of TS 33.501 [2] cannot be used and if the employed EAP method supports privacy.

Editor's note: What solution to use to identify KTNGF in step 13 is FFS.

- 	Support for all key generating EAP-methods: Extension of applicable authentication mechanism in step 8 to key-generating EAP authentication methods.

- 	Support for onboarding: Add possibility to send onboarding SUCI in step 5

Editor’s Note: Further conclusions are FFS.

### 7.1.4 Conclusion for N5CW device access to SNPN

Solution #4 is selected as basis for normative work with regards to the aspects:

- 	Support for all key generating EAP-methods

- 	Support for usage of anonymous SUCI if the construction of SUCI as described in clause 6.12 of TS 33.501 [2] cannot be used and if the employed EAP method supports privacy.

- 	Support for SNPN Id (PLMN Id and NID) carried in NAI

Editor’s Note: Conclusions regarding the issue of key derivation for non-NAS capable devices shall be aligned with outcome of study in TR 33.887 and are FFS.

Editor’s Note: Further conclusions are FFS.

### 7.1.5 Conclusion for NSWO support in SNPN

The solutions support usage of anonymous SUCI if the construction of SUCI as described in clause 6.12 of TS 33.501 [2] cannot be used and if the employed EAP method supports privacy.

Solution #9 is selected as basis for normative work with respect to the aspects of supporting NSWO in SNPN that has AUSF/UDM.

How the UDM selects authentication method in case of anonymous SUCI is to be specified as part of normative work.

Solution #14 is selected as basis for normative work with respect to the aspects of supporting NSWO in SNPN using a  CH with AUSF/UDM.

Solution #15 is selected as basis for normative work with respect to the aspects of supporting NSWO in SNPN using SNPN credentials from CH AAA.

## 7.2 	Conclusions for KI#2 Authentication for UE access to hosting network

Existing authentication procedures can be used for authentication for UE access to hosting network in the possible scenarios as concluded in TR 23.700-08 [2].

Editor's Note: Whether the usage of existing authentication methods in the context of providing localized services can be described in informative text is ffs.

Editor's Note: Whether additional security measures are necessary is ffs.

***END CHANGES***