3GPP TSG-SA3 Meeting #110 	S3-231506r4

# Athens, Greece, 20 - 24 February 2023						(mergerof S3-230849 and S3-230915)

# Source:	Huawei, HiSilicon, ZTE

# Title:	Update to KI#1

# Document for:	Approval

Agenda Item:	5.12 FS_eNS_Ph3

# 1	Decision/action requested

Approve the update to KI#1 in TR33.886

# 2	References

[1]

# 3	Rationale

The contribution proposes to update KI#1 implementing part of the way forward document S3-230848.

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## 4.1	Key Issue #1: providing VPLMN slice information to roaming UE

### 4.1.1	Key issue details

The following requirement for a 5G network is specified in TS 22.261[2] in order to support a roaming UE activating network slice services

For a roaming UE activating a service/application requiring a network slice not offered by the serving network but available in the area from other network(s), the HPLMN shall be able to provide the UE with prioritization information of the VPLMNs with which the UE may register for the network slice

A related key issue is also being studied in TR 23.700-41 [3] for possible procedure changes to automatic PLMN selection for a roaming UE requiring a network slice not offered by the serving network but available in the area from other network(s). It is expected that the corresponding security procedure will be affected (e.g. Steering of Roaming (SoR) in TS33.501 [4]) in order to support the HPLMN to provide a roaming UE the enhanced slice-aware information which includes preferred PLMNs for specific S-NSSAIs in the UE subscription.

In this key issue, the following aspects will be studied:

-	Would security procedures be impacted? If so which security procedures are impacted in support of HPLMN proving a roaming UE with information about prioritization information of the VPLMNs with which the UE may register for the network slice?

- 	How to secure the procedures impacted.

### 4.1.2	Security threats

If slice-aware SoR information is not integrity protected, it may be tampered by the VPLMN. If the information is sent to an unauthorized UE, it may cause leakage of the information.

Compared to existing services where Steering of Roaming procedure is used to protect in integrity the data, this Key Issue does not bring any additional security threat if the slice-aware SoR information is included in the protected data.

### 4.1.3	Potential security requirements

The 5G system shall provide integrity protection to the enhanced slice-aware SoR information and the existing SoR mechanism defined in TS 33.501 shall be used for protecting in integrity slice-aware SoR information.

***	END OF CHANGES	***