3GPP TSG-SA3 Meeting #110	S3-231516

# Athens, Greece, 20 - 24 February 2023

# Source:	Huawei, HiSilicon

# Title:	New solution for protecting Ranging unicast communication

# Document for:	Approval

Agenda Item:	5.19

1	Decision/action requested

Approve the new solution proposal to TR 33.893

2	References

N/A

3	Rationale

The contribution proposes to add a new solution to provide secure direct communications.

Revision of S3-231029.

4	Detailed proposal

***BEGINNING OF THE 1st CHANGE***

6.X	Solution #X: Security of Ranging unicast communication

6.X.1	Introduction

This solution addresses the protection of direct communication (unicast) for Ranging/SL Positioning service as specified in Key Issue #4. Generally, the solution uses the direct communication security defined for 5G ProSe in TS 33.503 [6] and/or for 5G V2X in TS 33.536 [5] as a baseline and adjusts to the Ranging/SL Positioning scenario.

6.X.2	Solution details

In this solution, the Ranging/SL positioning capable UEs are provisioned with the list of Ranging/SL Positioning services, with Geographical Area(s) and their PC5 security policies. The definition of the PC5 security policies follows the definition in 5G eV2X in TS 33.536 [5] and/or in ProSe in TS 33.503 [6].

During the establishment of the direct communication for the Ranging/SL positioning service, the UEs use the PC5 security policy to negotiate the final security protection status, as defined in 5G ProSe in TS 33.503 [6] and/or in 5G V2X in TS 33.536 [5]. The Ranging/SL Positioning signalling are protected based on the PC5 UP security policies.

Edirot’s Note: It’s FFS whether a PC5 link sharing multiple ranging/SL Positioning services.

6.X.3	Evaluation

This solution addresses the first three security requirements of Key Issue #4. The mutual authentication between two UEs during one-to-one communication is supported by reusing the PC5 security establishment procedure in 5G ProSe in TS 33.503 [6] and/or in 5G V2X in TS 33.536 [5].

Edirot’s Note: further evaluation is FFS.

***END OF THE CHANGES***