3GPP TSG-SA3 Meeting #110 	 S3-231610

# Athens, Greece, 20 - 24 February 2023 										Revision of S3-230865

# Source:	Huawei, HiSilicon

# Title:	Add test case to include SNPN snenario in token verification

# Document for:	Approval

Agenda Item:	4.4

1	Decision/action requested

Approve the new test case for inclusion in living doc of 33.117.

2	References

NA

3	Rationale

It's proposed to add the test case to address the agreed threat in TR 33.926 related to SNPNs with Credentials Holder using AUSF and UDM for primary authentication. The only difference between the test case for SNPNs with Credentials Holder using AUSF and UDM for primary authentication and PLMNs is the verification of SNPN ID and PLMN ID.

4	Detailed proposal

It is proposed to approve the following changes for inclusion in the living doc of 33.117.

**** Start of Changes****

4.2.2.2.3.2	Authorization token verification failure handling in different PLMNs or SNPNs

Requirement Name: Authorization token verification failure handling in different PLMNs or SNPNs

Requirement Reference: TS 33.501 [10], clause 13.4.1.2

Requirement Description:

"The NF service producer shall check that the home PLMN ID of audience claim in the access token matches its own PLMN identity.

For SNPNs with Credentials Holder using AUSF and UDM for primary authentication, the NF Service Producer verifies the SNPN ID of the serving SNPN contained in the API request instead of the PLMN-ID, and the SNPN ID or the PLMN ID of the Credentials Holder instead of the home PLMN ID"

Threat References:  TR 33.926 [4], clause 6.3.3.1, Incorrect Verification of Access Tokens

NOTE 1:	The test case below only applies to the NFs which support identifying and understanding the producerPlmnId claim or the producerSnpnId claim.

Test Case:

Test Name: TC_AUTHORIZATION_TOKEN_VERIFICATION_FAILURE_DIFF_PLMN_SNPN

Purpose:

Verify that the NF service producer does not grant service access if the verification of authorization token from a NF service consumer in a different PLMN or SNPN fails.

Procedure and execution steps:

Pre-Conditions:

-	Test environment with a NF service consumer and two SEPPs (one cSEPP, one pSEPP).

-	The NF service consumer and SEPPs may be simulated.

-	The network product under test has already mutually authenticated with the NF service consumer in a different PLMN or SNPN via the SEPPs.

-	The tester has the NRF’s private key or the shared key.

-	The network product under test is preconfigured with the NRF’s public key or the shared key.

-	The tester shall have access to the interfaces of the NF service consumer and the network product under test.

NOTE X:	The SNPN ID related test case below only applies to UDMs, NRFs and AUSFs in Credentials Holders using AUSF and UDM for primary authentication.

Execution Steps

The network product under test receives the access token sent from the NF service consumer, verifies the access token in accordance with the execution steps in 4.2.2.1.2.1, with the following additional test cases:

Test Case 1: incorrect PLMN ID or SNPN ID of the NF service producer in the access token

1)	The test computes an access token correctly, except that the PLMN ID in the producerPlmnId claim or the SNPN ID in the producerSnpnId claim of the access token is empty or different from the home PLMN ID or SNPN ID of the network product under test, and then includes the access token in the NF Service Request sent from the NF service consumer to the network product under test through the SEPPs.

2)	The network product under test receives the access token sent from the NF service consumer through the SEPPs, verifies that the PLMN ID in the producerPlmnId claim or the SNPN ID in the producerSnpnId claim of the access token is different from its own home PLMN or SNPN identity.

Test Case 2: absent PLMN ID or SNPN ID of the NF service producer in the access token

1)	The test computes an access token correctly, except that no producerPlmnId or producerSnpnId claim is included in the access token, and then includes the access token in the NF Service Request sent from the NF service consumer to the network product under test through the SEPPs.

2)	The network product under test receives the access token sent from the NF service consumer through the SEPPs, verifies that the access token is not a token to be used by the NF service consumer in a different PLMN or SNPN, based on the absence of PLMN ID or SNPN ID of the NF service producer in the access token.

Expected Results:

For both test cases 1 and 2, the network product under test rejects the NF service consumer’s service request based on Oauth 2.0 error response defined in RFC 6749 [12].

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

**** End of Changes****