3GPP TSG-SA3 Meeting #110 	S3-231614

# Athens, Greece, 20 - 24 February 2023

# Source:	Nokia, Nokia Shanghai Bell

# Title:	New SID on security for N32 and SEPP hosted scenarios

# Document for:	Approval

# Agenda Item:	6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on security for N32 and SEPP hosted scenarios

Acronym:	FS_N32SEPP_SEC

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | x | x | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

N/A

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| 5GS_Ph1-SEC | SA3 | 750016 | Security aspects of 5G System - Phase 1 |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

SA3 has received from GSMA two liaison statements S3-213806 and LS S3-221737. GSMA progress is pending on SA3 specified 5G N32 security and requirements on SEPP in 3GPP. In summary:

- S3-213806: N32 security in mediated roaming scenarios

The GSMA is working on best practice guidance and related specifications for how operators interact with both IPX and roaming hubs. However, the pre-5G architecture of these methods uses the 4G hop-by-hop security paradigm.

In 5G, SEPP communication for roaming relies on end-to-end security by design as specified in TS 33.501, i.e., messages are authenticated and integrity-protected between the SEPP of the visited network and the SEPP of the home network. This creates the need to study in more detail how the migration away from hop-by-hop security can be achieved without loss of the benefits provided by IPX providers and roaming hubs while the 5G security requirements on N32 are met.

- S3-221737: Security in hosted SEPP scenarios

In some scenarios, the operator may decide to outsource the operation of its SEPP to an external entity. This scenario is called the “Hosted SEPP” scenario. The Hosted SEPP scenario introduces security requirements which must be fulfilled by N32 and by the connection between PLMN and Hosted SEPP.

The decision of an operator to outsource the operation of a SEPP to an external entity can be independent of whether and which IPX providers are used, and whether and which roaming hub(s) are used. In case an operator uses a Hosted SEPP, the security perimeter of the PLMN as described in TS 33.501, clause 4.2.1, extends to an entity external to the PLMN.

# 4	Objective

The study will focus on security for N32 and SEPP hosted scenarios for 5G roaming raised via liaisons with GSMA.

Identified topics are:

- N32 security in mediated roaming scenarios

- Security in hosted SEPP scenarios

# 5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | 33.XXX | Study on security for N32 and SEPP hosted scenarios | TSG#101 | TSG#103 | Jerichow, Anja Nokia anja.jerichow(at)nokia.com |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Jerichow, Anja (Nokia): anja.jerichow@nokia.com

# 7	Work item leadership

SA3

# 8	Aspects that involve other WGs

The result of this study can have impact to CT4 specifications.

# 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell

Deutsche Telekom

Mavenir

Huawei

HiSilicon

Ericsson

Vodafone