3GPP TSG-SA3 Meeting #110	S3-231616

# Athens, Greece, 20 - 24 February 2023

# Source:	Qualcomm Incorporated

# Title:	Updates on the solution #24

# Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes to update the solution #24 in TR 33.740.

# 2	References

[1]	TS 33.503

[2]	TR 33.740 v0.5.0

# 3	Rationale

This contribution proposes to update the solution #24.

In particular, this contribution proposes to provision direct discovery set protection indication to source UE and target UE when an RSC and the corresponding security materials are provisioned at those UEs. The direct discovery set protection indication informs the UE of whether to enable/disable the direct discovery set(s) protection, thus, enables a source UE/ target UE to determine whether to do/undo the protection of direct discovery set(s) during the U2U relay discovery procedure. The direct discovery set protection indication is set to be enabled when the U2U relay service identified by the RSC supports to relay multiple direct discovery set(s) associated with different ProSe services. Otherwise, it is set to be disabled.

In addition, since the solution clarified when two sets of discovery security materials are needed and used, the below two Editor’s Notes are no longer needed. Thus, it is proposed to delete the below two Editor’s Notes.

Editor’s Note: The need of two sets of security parameters is FFS.

Editor’s Note: It's FFS whether ProSe services requiring different security materials share the same RSC.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [2].

***** START OF CHANGES *****

## 6.24	Solution #24: Security mechanism for UE-to-UE Relay Model B discovery

### 6.24.1	Introduction

This solution addresses the Key Issue #1.

A (relayed) Solicitation message and (relayed) Response message by the U2U relay includes two sets of elements, i.e., direct discovery set(s) (e.g., the User Info ID of source UE and target UE) and a U2U discovery set (e.g., Type of Discovery Message, RSC, User Info ID of the relay). The 5G ProSe U2U Relay only modifies the elements of the U2U discovery set. The direct discovery set that is constructed by the source UE can only be interpreted by the target UE, and vice versa. The U2U Relay can use a single RSC (and the associated security materials) to relay direct discovery sets associated with the multiple ProSe services. This means that the 5G ProSe U2U Relay uses the same security materials to protect the discovery messages that contain the direct discovery sets associated with different ProSe service. Unless each direct discovery set is protected using the security materials of the corresponding ProSe service, a source UE (or a target UE) that is authorized to use the RSC for U2U discovery can decrypt (and manipulate) any direct discovery sets that are delivered using the same RSC. This poses security threats.

To prevent such security threats, this solution supports both scenarios by provisioning direct discovery set protection indication to the source UE and target UE. In case a single RSC is used to relay multiple direct discovery sets associated with different ProSe services, the discovery messages are protected using two sets of security materials (i.e., the discovery messages and the direct discovery set(s) are protected using the respective set of security materials).

In case a single RSC is used to relay direct discovery sets associated with a single ProSe service, the discovery messages are protected using a single set of security materials associated with the RSC.

### 6.24.2	Solution details

Figure 6.24.2-1: Model B discovery

This solution consists of two protection mechanisms using two sets of security materials: one for the Solicitation and Response message protection and the other one for the direct discovery set(s) protection.

Editor’s Note: The need of two sets of security parameters is FFS.

Editor’s Note: It's FFS whether ProSe services requiring different security materials share the same RSC.

The security procedure is described as follows:

NOTE: the procedures for discovery security materials provisioning and protection mechanisms specified in clause 6.1.3.2 of TS 33.503 [6] are reused for the U2U Relay Model B discovery. The details of how the protection mechanisms in clause 6.1.3.2.3 of TS 33.503 [6] is applied to the U2U discovery message protection will be specified during the normative work.

0.	The source UE, target UE, and U2U Relay are provisioned with the following discovery security materials based on the procedure specified in clause 6.1.3.2.2 of TS 33.503 [6].

0a.	The source UE and target UE are provisioned with the discovery security materials associated with the ProSe code (i.e., query and response code) for a ProSe service.

0b.	The source UE, target UE, and U2U Relay are provisioned with discovery security materials associated with an RSC. In addition, the source UE and target UE are provisioned with the direct discovery set protection indication informing whether to enable or disable the direct discovery set(s) protection.

Editor’s Note: The need of two different discovery codes (ProSe code and RSC) is FFS.

1.	The source UE protects a direct discovery set(s) (e.g., user info ID of target UE) using the discovery security materials associated with the ProSe code if the direct discovery set protection indication is set to be enabled. Then, the source UE provides the protected direct discovery set(s) to the U2U Relay via a Solicitation message. The Solicitation message is protected as specified in clause 6.1.3.2.3 of TS 33.503 [6].

2.	On receiving the Solicitation message from the source UE, the U2U Relay decrypts and/or verifies the received Solicitation message using the discovery security materials associated with the RSC. If the verification is successful, the U2U Relay constructs a (relayed) Solicitation message that contains U2U discovery set and the direct discovery set(s). Then, the U2U Relay protects the (relayed) Solicitation message using the same security materials and forwards (relays) the message to the target UE.

3.	The target UE decrypts and/or verifies the received Solicitation message using the discovery security materials associated with the RSC. If the verification is successful and the direct discovery set protection indication is set to be enabled, the target UE further extracts the protected direct discovery set(s) from the message and decrypts and/or verifies the direct discovery set(s) using the discovery security materials associated with the ProSe code.

4.	The target UE protects direct discovery set(s) (e.g., user info ID of source UE) using the discovery security materials associated with the ProSe code if the direct discovery set protection indication is set to be enabled and includes it in a Response message. Then, the target UE protects the Response message using the discovery security materials associated with the RSC and sends the message to the U2U Relay.

5.	The U2U Relay decrypts and/or verifies the received Response message using the discovery security materials associated with the RSC. If the verification is successful, the U2U Relay constructs a (relayed) Response message that contains (modified) U2U discovery set and the direct discovery set(s) and protects the message using the same security materials. Then, the U2U Relay forwards (relays) the message to the source UE.

6.	The source UE decrypts and/or verifies the received Response message using the discovery security materials associated with the RSC. If the verification is successful and the direct discovery set protection indication is set to be enabled, the source UE further extracts the protected direct discovery set(s) from the message and decrypts and/or verifies the direct discovery set(s) using the discovery security materials associated with the ProSe code.

### 6.24.3	Evaluation

This solution addresses the Key Issue #1. In particular, this solution supports both scenario s by provisioning direct discovery set protection indication to the source UE and target UE:

- Two sets of discovery security materials are used to protect direct discovery set and the discovery messages in case a single RSC is used by a 5G ProSe U2U Relay to relay multiple direct discovery sets associated with different ProSe services.

- A single set of discovery security materials is used to protect the discovery messages in case a single RSC is used by a 5G ProSe U2U Relay to relay direct discovery set(s) associated with a single ProSe service.

Editor’s Note: Further evaluation is FFS.

***** END OF CHANGES *****