3GPP TSG-SA3 Meeting #111 	S3-232639

# Berlin, Germany, 22 -26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | draftCR | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add AKMA roaming requirements and network model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 990040 |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add AKMA roaming requirements and network model as studied in TR 33.737 |  |  |  |  |  |  |  |  |
| Summary of change: |  | AKMA roaming requirements and network model |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete contents |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 4.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF 1st CHANGE ****

4.X	Roaming

4.X.1	AKMA roaming requirements

Depending on UE and AF locations, the AKMA roaming network model shall support 3 different AKMA roaming scenarios:

-	UE is in VPLMN and accessing an internal HPLMN AF;

-	UE is in VPLMN and accessing an internal VPLMN AF;

-	UE is in VPLMN and accessing an external AF in the Data Network.

4.X.2	AKMA roaming network model

Figure 4.X.2-x shows the fundamental roaming network model for the case that the UE is roaming in the VPLMN and accesses an AF internal to the HPLMN. This network model is a simplified version of the Roaming 5G System architecture - home routed scenario from TS 23.501 [3] since the UE accesses an AF internal to the HPLMN. It is therefore assumed that the UE is configured to setup a home routed PDU session for the Ua* protocol between itself and the AF. The HPLMN AF accesses the AAnF following the internal AF model.

# Figure 4.X.2-x: Fundamental Roaming Network Model for AKMA when the AF is internal to the HPLMN

Figure 4.X.2-y shows the fundamental roaming network model for the case that the UE is roaming in the VPLMN and accesses an AF internal to the VPLMN or external in the Data Network. This network model is a combination and simplification of the Roaming 5G System architecture - local breakout and home-routed scenarios from TS 23.501 [3]. In the case that the UE accesses an internal AF to the VPLMN a local-breakout scenario is used between the UE and AF. In the case that a UE accesses an external AF which is in the DN, it does not matter if the UE uses the local breakout or the home-routed scenario. A VPLMN AF and an external AF accesses the AAnF via the HPLMN NEF following the external AF model. In other words, the N32 interface is not used in the case of the AF internal to the VPLMN.

Figure 4.X.2-y: Fundamental Roaming Network Model for AKMA when the AF is internal to the VPLMN or AF is external

**** END OF 1st CHANGE ****