3GPP TSG-SA3 Meeting #112 	S3-233590

# Goteborg, Sweden, 14 - 18 August 2023

# Source:	Philips International B.V.

# Title:	Procedure for secure groupcast/broadcast of SL positioning signalling

# Document for:	Approval

Agenda Item:	4.26

# 1	Decision/action requested

SA3 is kindly requested to approve the contributed text describing requirements and the procedure for secure groupcast/broadcast of SL positioning signalling

# 2	References

# 3	Rationale

Ranging requires the groupcast/broadcast of SL positioning signalling as described in KI#5 in TR 33.893. This contribution

- proposes to include related requirements in TS 33.533

- describes how to protect said signalling data (addressing KI#5 in TR 33.893) by applying LTE positioning procedures described in TS 38.305 [12] (Clause 7.5), TS 23.273 [9] (Clause 6.14.2) and TS 37.355 [13] (Clause 7.5).

# 4	Detailed proposal

*************** Start of changes ****************

### 6.4.2       Security requirements

The Ranging/SL Positioning system shall support mutually authentication between the UEs during unicast direct communication establishement for Ranging/SL Positioning control over RSPP.

The Ranging/SL Positioning system shall support integrity, confidentiality and anti-replay protection for the information transferred during unicast direct communication for Ranging/SL Positioning control over RSPP.

The Ranging/SL Positioning system shall support cryptographic separation for each SR5 interface and for each peer UE during unicast direct communication for Ranging/SL Positioning control over RSPP.

The Ranging/SL Positioning system shall support integrity, confidentiality and anti-replay protection for the information transferred during unicast communication for Ranging/SL Positioning control over the protocol between the UE and LMF.

The Ranging/SL Positioning shall support confidentiality, integrity and replay protection of SL positioning signalling during groupcast communication.

The Ranging/SL Positioning shall support confidentiality, integrity and replay protection of SL positioning signalling during broadcast communication.

The Ranging/SL Positioning system shall mitigate trackability and linkability attacks of the UE during groupcast communication for Ranging/SL positioning.

The Ranging/SL Positioning system shall mitigate trackability and linkability attacks of the UE during broadcast communication for Ranging/SL positioning.

*************** Start of next change ****************

## 6.4.X	Security for groupcast/broadcast of SL positioning signalling

### 6.4.X.1	Security procedure

Security for groupcast/broadcast of SL positioning signalling builds on the positioning-related security procedures described in TS 38.305 [12] (Clause 7.5), TS 23.273 [9] (Clause 6.14.2) and TS 37.355 [13] (Clause 7.5)

Figure 6.X-1 Procedure for secure broadcast/groupcast of SL positioning signalling data

The procedure for secure broadcast/groupcast of SL positioning signalling data is shown in Figure 6.X-1 and involves the following steps:

In Step 1, the UEs (in the following, UEs refer to UE1 or UE2) send a Registration Request (similar to TS 23.273 [9], Clause 6.14.2)). In Step 2, the UEs are configured by means of a Registration Accept (as in TS 23.273 [9], Clause 6.14.2), with the corresponding security information as applicable to the ranging service, if the UEs are authorized.

If devices are in-coverage / partial coverage, Steps 3-7 are applicable. Steps 8-9 may apply based on a configuration. In Step 3, the UEs determine a ranging trigger that requires the distribution of SL ranging data when in-coverage (IC) or partial coverage (PC). In Step 4, when a ranging trigger occurs, UE2 sends a ranging request to retrieve SL positioning signalling data. In Step 5, the CN, e.g., LMF, determines suitable SL positioning data, and protects it as in TS 37.355 [13]. In Step 6, the protected SL positioning signalling data is transferred to UE2 in charge of local groupcast/broadcast. This message is securely transferred by means of NAS protection. In Step 7, UE2 may verify the received SL positioning signaling data before UE2 broadcasts/groupcasts it in Step 10.

If devices are out of 3GPP coverage, Steps 8-9 are applicable. The UE responsible for groupcast/broadcast implements the functionality of server UE, thus replacing LMF functionality. In Step 8, the UEs determine a ranging trigger that requires the distribution of SL positioning signalling data when out of 3GPP coverage. Then, in Step 9, when a ranging trigger occurs, UE2 protects SL positioning signalling data to be distributed in Step 10.

In Step 10, UE2 groupcasts/broadcasts the message including the protected SL positioning signalling data. UE1 receives and securely processes the message containing the protected SL positioning signalling data.

### 6.4.X.2	 Keys and algorithms for groupcast/broadcast of SL positioning signalling

UEs may be configured with one or more of the following keys in Step 2:

- a CipherSetID as in TS 37.355 that identifies the keys used for groupcast/broadcast. The CipherSetID is bound to the group of UEs sending/receiving the groupcast/broadcast communication.

- a key Kc_ic as in TS 37.355 [13] for confidentiality protection when UEs are IC/PC.

- a key Kc_ooc as in TS 37.355 [13] for usage when UEs are OOC. In this case, UE2 uses Kc_ooc in step 9 for confidentiality protection of SL positioning data.

- a key Ki_ic for the computation of a MIC that allows e.g., UE1 to verify SL positioning signaling data transmitted in groupcast/broadcast.

- a key Ki_ooc for the computation of a MIC for usage when UEs are OOC. In this case, UE2 uses Ki_ooc in step 9 for integrity protection of SL positioning data.

- a key Ks_ic for scrambling the CipherSetID when UEs are IC.

- a key Ks_ooc for scrambling the CipherSetID when UEs are OOC. In this case, UE2 uses Ks_ooc in step 9 for scrambling the CipherSetID.

The message structure in Step 10 includes the following fields:

- ExpirationDate (how long the message is valid)

- ValueTag (whether the message changed since the last message)

- CipherSetID (identifier used to identify the keys used for integrity/confidentiality protection)

- Counter value (a counter used for encryption)

- SL positioning signaling data (data that is confidentiality protected)

- MIC (protecting integrity of the message)

When a UE sends a groupcast/broadcast message with SL positioning signaling data associated to CipherSetID, the UE processes the message in Steps 6 and 9 as follows:

- When integrity keys are available, integrity protection in Steps 6 and 9 is done by including a message integrity code (MIC) in the message. The MIC is computed using as input the SL positioning signaling data as well as the fields valueTag and expirationDate, as defined in TS 37.355.

- When confidentiality keys are available, confidentiality protection in Steps 6 and 9 is done as in TS 37.355 [13] using the corresponding confidentiality key. Confidentiality protection is applied to the SL positioning signaling data that corresponds to the AssistanceDataSIBelement in TS 37.355, and requires a cipherSetIDthat identifies a cipher set comprising a cipher key value and the counter value, as in TS 37.355.

- When a scrambling key is available, the CipherSetID is scrambled. Scrambling is performed by XORing the CipherSetID with the output of the KDF taking as input the scrambling key and the UTC-based counter setting the b least significant bits to zero.

When a UE receives a protected groupcast/broadcast message with SL positioning data, the UE processes the message in Step 10 as follows:

- When a scrambling key is available, the CipherSetID is unscrambled. The UE determines if the  groupcast/broadcast message with SL positioning signaling data is addressed to the UE group based on whether the CipherSetID matches.

- When a confidentiality key is available, the SL positioning signaling data is decrypted according to TS 37.355.

- When an integrity key is available, the integrity of the SL positioning signaling data is verified. The UE rejects the message if the expirationDate is in the past.

*************** End of changes ****************