3GPP TSG-SA3 Meeting #112 	S3-233615

# Goteborg, Sweden, 14 - 18 August 2023

Title:	Draft_LS reply for R2- 2306693 on Reporting of Relay UE C-RNTI and NCGI

Response to:	R2- 2306693 on Reporting of Relay UE C-RNTI and NCGI

Release:	Release 18

Work Item:	NR_SL_relay_enh-Core

Source:	SA3

To:	RAN2

Cc:

Contact person:

Name: 		Jing Ping

Email Address:	jing.ping@nokia-sbell.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None	-

1. Overall Description:

SA3 thanks RAN2 for their LS on Reporting of Relay UE C-RNTI and NCGI.

RAN2 asks SA3 to check if any security concern on the working assumption (WA) that remote-UE reports the RRC_CONNECTED relay-UE C-RNTI and serving cell ID (e.g., NCGI) for indirect path addition during the operation of multi-path in Scenario 2 (where the remote UE accesses the network via both a direct Uu path and another indirect relayed path which is an ideal UE-UE connection but not defined in 3GPP). This WA indicates that the C-RNTI and serving cell ID (e.g., NCGI) of relay UE is visible to remote UE during the operation of multi-path in Scenario 2.

SA3 analyzed the scenario and confirms there is no security concern on reporting of C-RNTI and serving cell ID of relay UE by the remote UE with following justifications:

- Trust relationship between remote UE and relay UE is established when relay UE has been decided to provide relay service to the remote UE, e.g. relay UE already authenticated remote UE.

- Similarly, security link could be established between remote UE and relay UE after the relay UE authenticated the remote UE.

- The AS security is available to protect C-RNTI and serving cell ID of the relay UE in report from remote UE to gNB through the direct path.

2. Actions:

To SA3

ACTION: 	SA3 would like to respectfully ask RAN2 to take above information into account.

3. Date of Next TSG-SA WG3 Meeting:

SA3#113	6 -10 November 2023	Chicago, US

SA3#114	22 -26 January 2024	UE (TBD)