3GPP TSG-SA3 Meeting #112	S3-233669

# Goteborg, Sweden, 14 - 18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1697 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NSWO support in SNPN using CH with AAA server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CableLabs, Charter Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-07-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add the option for NSWO procedure to be executed between the UE and the CH AAA server via NSWOF and 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the procedure for NSWO authentication between the UE and CH AAA server via NSWOF and 5GC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unauthorized access to operator’s WLAN access network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex I.10.5.1, Annex I.10.5.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** 1st CHANGE ***

## I.10.5	Security for NSWO support in SNPN

### I.10.5.1	NSWO support in SNPN using CH with AAA server via AAA Proxies

The reference architecture to support authentication for Non-seamless WLAN offload using SNPN credentials from Credentials Holder using AAA Server is specified in Figure 4.2.15-3b of TS 23.501 [2].

This clause applies to UEs that support NSWO in SNPN with any key generating EAP method by using the SNPN credentials with CH AAA server (i.e., the MSK indication described in step 0 of clause I.2.2.2.2 is configured on the UE). The UE shall reuse 5G NSWO procedures defined in Annex S.3.2 with WLAN access network for NSWO authentication with the same key generating EAP method that is used by the SNPN over 3GPP access with the following exception:

-	 The EAP authentication is performed between the UE and the CH AAA with no involvement of NSWOF/AUSF/UDM.

-	The construction of SUCI described in this document is not applicable.

NOTE: The requirement to use the same key generating EAP method includes the EAP identity privacy mechanism used by the SNPN over 3GPP access. This implies that for NSWO support in SNPN using CH with AAA server the identity privacy is achieved at the EAP layer via EAP method specific means.

### I.10.5.2	NSWO support in SNPN using CH with AAA server via 5GC

Figure I.10.5.2-1 - Procedure for NSWO authentication using CH with AAA server via 5GC

1-4. Same as steps 1-4 of clause S.3.2, except that SUCI is replaced with NAI. If the EAP method supports privacy and the UE is configured to use anonymous NAI, the UE may send an anonymous NAI.

5. Same as step 5 of clause S.3.2, except that NSWOF constructs the SUCI from the NAI received in step 4.

6-10. Same as steps 3-7 of clause I.2.2.2, except that an NSWO_indictor is also sent to the UDM by the AUSF.

11. Same as step 8 of clause I.2.2.2, where the UE and the AAA server performs primary authentication using any key-generating EAP method.

12-13. Same as steps 9-10 of clause I.2.2.2.

Remaining steps are performed as described in steps 16-18d of clause S.3.2.

*** END OF CHANGES ***