3GPP TSG-SA3 Meeting #112 	S3-233699

# Goteborg, Sweden, 14 -18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1715 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Address the EN for handing 2 AMFs problem |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | HN_Auth |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In multi-registration case, when the authentication associated with one AMF is pending, the UDM may select another AMF after receiving an acknowledgement. And the double authentication problem also needs to be handled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Address the EN and update 6.1.5.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************* Start of 1st Change *************

6.1.5.2	Security mechanisms

The UDM may initiate primary authentication based on procedures initiated by the UE (e.g. UE registration in 5GC) or towards the UE (e.g. SoR/UPU) or events from other NFs, considering the local policy into account as well.

Figure 6.1.x.2-1 Home Network triggered primary authentication procedure

0a.[optional] The UDM may be pre-configured with an operator authentication  policy in order to determine when to trigger a primary authentication procedure.

0b.	The UE registers to the network. As part of the registration, the serving AMF registers the UE with the UDM via the Nudm_UECM_Registration as per TS 23.502 [8], clause 4.2.2.2.2. The UDM shall create an implicit subscription for the serving AMF for the UDM to later notify the AMF for potential re-authentication.

1. A prerequisite for the initiation of Home Network Triggered Primary Authentication is that the UDM already has the information about the AMF serving the UE. Otherwise, the UDM cannot contact any AMF in later steps.

The UDM decides itself based on events (e.g., SoR/UPU or if AAnF requests) or authentication policy and performs home network triggers primary authentication as described in the following steps. The AAnF considers based on certain factors to send Nudm_UECM_AuthTrigger request to the UDM for primary authentication using the UDM services as described in clause 14.2.X.

Editor’s Note: The factor the AAnF considers to request UDM for primary authentication is FFS, as AAnF request(s) should not lead to signalling overload as well as the AF key expiry/refresh issue should be handled.

Based on a received event and the local operator authentication policy, if there is no ongoing primary authentication for the UE, and if the UDM determines to trigger the primary authentication, the UDM determines the serving AMF/SEAF of the target UE.

If there are different AMFs registered in the UDM for different access, and the procedure defined in 6.1X is supported, the UDM shall select one AMF to perform the reauthentication. The criteria for selecting the AMF are dependent of the local UDM authentication policy.

NOTE 1:	The reasons for the UDM determining that the UE needs to be authenticated can be different. For example, the UDM can determine to initiate a primary authentication when the AMF registers the UE upon the Registration procedure during the mobility from EPC or when SoR/UPU counters are about to wrap around, or when required based on authentication policy, or based on the request from AAnF. The UDM behaviour is determined by operator  policy which takes into account the support of certain features in the PLMN. For example, if the PLMN does not support the SoR/UPU feature, then SoR/UPU counter wrap around will not happen and primary authentication will not be required for this case.

2. The UDM sends a notification message to the AMF/SEAF with the UE’s SUPI.

Editor’s Note: The name of notification message between AMF and UDM is FFS.

3. After receiving the notification  message from the UDM, the AMF/SEAF shall decide whether run the primary authentication procedure based on its own local authentication policy, and the UE state (e.g. if the UE is under handover, a similar way as some steps in Network Triggered Service Request (TS 23.502 [8], clause 4.2.3.3) can be reused, or if the UE is already under authentication by the AMF before receiving the authentication notification from the UDM). If the AMF/SEAF cannot run a primary authentication as described in steps 4, the AMF/SEAF sends the authentication response message to the UDM with a  failure cause else it acknowledges the request. If the AMF/SEAF acknowledged the request but the AMF/SEAF is not able to initiate the primary authentication towards the UE (e.g. if UE is not reachable), the AMF/SEAF shall acknowledge the request with a pending indication and set the authentication pending flag in the UE context.

When UE re-attaches to the same AMF or becomes reachable, the AMF checks the authentication pending flag and checks with the UDM whether it can perform primary authentication for the UE if needed. The UDM may response according to local authentication policy. Once UE reauthentication is done, the AMF resets the authentication pending flag. If the UE attaches to a new AMF (e.g., in a mobility scenario), then the new AMF may retrieve the UE Context from the previous AMF, determine that authentication is pending for the UE, and perform reauthentication.

Upon receiving a failure or a pending indication from the AMF, the UDM may check if another AMF is available over the other access. If available, the UDM may select another AMF and send an authentication request. If one AMF is able to do the primary authentication for the UE, the UDM sends a message to the other AMF indicating temporarity not to perfom primary authentication even when the UE re-attaches to the other AMF to avoid double authentication. Once the priamry authentication performed by one AMF is done, the UDM notifies the other AMF to reset the pending flag in the UE context.

4. The AMF/SEAF starts the primary authentication procedure as defined in clause 6.2.1 of the present document.

The UDM may execute other procedures (e.g. SoR/UPU) depending on the reason that motivated the UDM triggered (re-)authentication procedure in step 1.

************* End of 1st Change *************