# 3GPP TSG-SA3 Meeting #112 	S3-233718

# Goteborg, Sweden, 14 - 18 August 2023	(revision of xx-yyxxxx)

# Source:	Huawei, HiSilicon

# Title:	Authorization for the SL Positioning Client UE

# Document for:	Approval

Agenda Item:	4.26

1	Decision/action requested

Approve the changes in TS 33.533.

2	References

[1]	TR 33.893

3	Rationale

Based on the conclusion on Key Issue #2 in TR 33.893, the procedure for authorization of UE for Ranging/SL positioning service exposure is added.

4	Detailed proposal

************ START OF CHANGES ************

6.3.X	Procedure for authorization of UE for Ranging/SL positioning service exposure

For Ranging/SL Positioning service exposure through the network via control plane (i.e. clause 6.7.1.2.3, TS 23.586 [11]), the SL Positioning Client UE authorization is reusing the MO-LR authorization as specified in TS 23.273 [9].   For Ranging/SL Positioning service exposure through PC5, authorization check of the client UE for the service request may be performed by verifying the client UE identifier locally both in UE1 and UE2 (as specified in clause 6.7.1.1 in TS 23.586 [2]) or by the network via the PKMF for ProSe capable UEs. To achieve this, the info of SL Positioning Client UE shall be included in the PC5 message (i.e. from the client UE to UE1 and from UE1 to UE2) or in the NAS message (i.e. from UE1 to AMF). If not authorized, the Ranging/SL Positioning service request shall be rejected.

************ END OF CHANGES************