# 3GPP TSG-SA3 Meeting #112 	S3-233722

# Goteborg, Sweden, 14 - 18 August 2023	(revision of xx-yyxxxx)

# Source:	Huawei, HiSilicon

# Title:	Removal of the editor’s note on MT-LR procedure

# Document for:	Approval

Agenda Item:	4.26

1	Decision/action requested

Approve the changes in TS 33.533.

2	References

[1]	TS 23.273

3	Rationale

Based on procedures of SL-MT-LR involving LMF specified in TS 23.273 [1], an LCS Client or AF is able to obtain Ranging/Sidelink Positioning location results for a group of n UEs (n≥2), i.e. UE1, UE2, ..., UEn. To protect the privacy, the authorization procedure can be enhanced. The editor’s note is removed accordingly.

4	Detailed proposal

************ START OF CHANGES ************

6.3.3	Procedure for authorization of AF/5GC NF for Ranging/SL positioning service exposure

For the authorization of the AF or 5GC NF for Ranging/SL Positioning service exposure, the SL-MT-LR procedure specified in TS 23.273 [3] is taken as the baseline. The authorization shall be performed towards all the n UEs (n≥2), i.e. UE1, UE2, ..., UEn in the request message.. If all of the UEs don’t grant permission for Ranging/SL Positioning exposure, the GMLC shall reject the service request from the AF/5GC NF. If only part of the UEs don’t grant permission, the GMLC shall not include these UEs’ identifications when invoking the Namf_Location_ProvidePositioningInfo service operation towards the AMF.

When receiving the Ranging/SL Positioning service request from the AF/5GC NF, the GMLC interacts with the UDM to check the UE privacy profile. The UE LCS Privacy Profile defined in clause 5.4.2 of TS 23.273 [3] is taken as the baseline for the UE privacy profile for Ranging/SL positioning services.

Editor’s Note: the details of the content of UE privacy profile for Ranging/SL positioning services is FFS.

The GMLC interacts with the AMF to request the ranging result of UEs, which may include an indication of a privacy related action. If the indicator of privacy check related action indicates that the UE must either be notified or notified with privacy verification, a notification invoke message is sent to the UE if the signalling connection established. However, if the Ranging/SL Positioning service is disallowed by the UE, or signalling connection establishment fails and UE notification (including UE notification with privacy verification) is required, the AMF shall provide failure response to the GMLC.

************ END OF CHANGES************