3GPP TSG-SA3 Meeting #112 	S3-233736

# Gothenburg, Sweden, 14 -18 August 2023										   (revision of S3-yyxxxx)

# Source:	Huawei, HiSilicon

# Title:	API invoker obtaining authorization from resource owner

# Document for:	Approval

Agenda Item:	4.28

1	Decision/action requested

Approve the changes for RNAA in the living document: draftCR of SNAAPPY

2	References

3	Rationale

The contribution proposes a general procedure for authentication and authorization for RNAA using OAuth 2.0 authorization code grant or PKCE.

4	Detailed proposal

*************** Start of 1st Change ****************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[3]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[4]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[5]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[6]	IETF RFC 7519: "JSON Web Token (JWT)".

[7]	IETF RFC 7515: "JSON Web Signature (JWS)".

[8]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[9]	Void

[10]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[11]	IETF RFC 7636: “Proof Key for Code Exchange by OAuth Public Clients”.

*************** End of 1st Change ****************

*************** Start of 2nd Change ****************

#### 6.5.Y2e.2	Authorization using Oauth flow

Editor's note: this clause describes authorization making use of the relevant oauth flow. Additional subsubsubclauses may be added for additional oauth flows.

6.5.Y2e.a	API invoker obtaining authorization from resource owner

Figure 6.5.Y2e.a-1 Procedure of Obtaining Resource owner Authorization

1. CAPIF-1e authentication and secure session establishment is performed as specified in subclause 6.3.1.

2. API invoker shall trigger resource owner client to request authorization code based on the service API information of requested service API. If the requested API uses authorization code grant, the authorization request shall include API invoker ID, scope, state, redirect URL. If the requested API uses PKCE, the request shall additionally include code_challenge and code_challenge_method.

3. The RO client authenticates to the CCF by establishing a TLS connection with the CCF.

4. The resource owner client shall send Authorization Request to the CCF. If the requested API uses authorization code grant, the message shall include response type, API invoker ID, redirect URL, scope, state. If the requested API uses PKCE, the request shall additionally include code_challenge and code_challenge_method.

5. Based on the operator's policy, mutual authentication between resource owner and CCF may be performed.

NOTE:	The authentication method between resource owner and CCF/AUF is up to operator policy (e.g. an already deployed mechanism, or AKMA, or GBA) and is out of SA3 scope.

After successful authentication, CCF retrieves resource owner’s authorization to allow API invoker to access to the its resource.

6. After having resource owner’s authorization, the CCF shall send Authorization Response message to the Resource owner client. The message shall include authorization code with redirect URL and state.

7. The Resource owner client shall send the authorization code to the API invoker.

8. The API invoker shall send Access Token Request to the CCF as per the OAuth 2.0 [4] specification based on the service API information of requested service API. If the requested API uses authorization code grant, the grant_type shall be set as “authorization_code”, the requested scope shall also include GPSI who owns the requested resource, and the message shall also include authorization code which is received from step 7. If the requested API uses PKCE, the request shall additionally include code_verifier.

9. After successful checking of authorization code and code_verifier (optional), ensuring that resource owner allows API invoker to access to its resource, the CCF shall send Access Token Response with a new access token to the API invoker. The claim of the token is defined in Annex C.2.2 with extension that the scope includes the GPSI. The token is able to restrict the API invoker to access a specific resource (e.g., location, QoS, PDN connectivity status) of the resource owner.

10. The API invoker authenticates to the AEF by establishing a TLS session with the API exposing function.

11. The API invoker shall initiate invocation of a 3GPP northbound API with the AEF to access the resource owner’s resource. The access token received from the CCF shall be sent along with the northbound API invocation request as per OAuth 2.0 [4].

12. The API exposing function shall validate the access token. The AEF verifies the integrity of the access token by verifying the CAPIF core function signature. If validation of the access token is successful, the AEF shall verify the API invoker's Northbound API invocation request against the authorization claims in access token as described in C.7.1, including checking whether GPSI in the request is complied with GSPI in the claims of the access token, and ensuring that the API Invoker has access permission for the requested resource of the resource owner. After successful verification of the access token and authorization claims of the API invoker, the requested northbound API shall be invoked and the appropriate response shall be returned to the API invoker.

NOTE X:		The definitions for scope, state, redirect URL, authorization code is described in RFC 6749 [4].

NOTE Y:		The definitions for code_challenge, code_challenge_method and code_verifier are described in RFC 7636 [xx].

*************** End of 2nd Change ****************

*************** Start of 3rd Change ****************

C.2.2	Token claims

The CAPIF ‘Method 3 - TLS with OAuth token’ access token shall convey the following claims as defined in IETF RFC 7519 [6] and IETF RFC 6749 [4].

Table C.2.2-1: Access token standard claims

| Parameter | Description |
| --- | --- |
| exp | REQUIRED. The expiration time of the access token. Implementers MAY provide for some small leeway, usually no more than a few minutes, to account for clock skew (not to exceed 30 seconds). |
| client_id | REQUIRED. The identifier of the API Invoker making the API request as previously established with the CAPIF Core Function through onboarding. |
| scope | REQUIRED. A string containing a space-delimited list, comprising of the following as scopes associated with this token: For general authorization: - List of Services per AEF (e.g. “AEF1:Service1,Service2,Service3,...,ServiceX; AEF2:Service1,Service2,Service3,...,ServiceZ”) For RNAA: - List of GPSIes of Services per AEF (e.g. “AEF1:Service1:GPSI1,...,ServiceX:GPSIx; AEF2:Service1:GPSI1, ,...,ServiceZ:GPSIz”) |

The ‘exp’and ‘scope’ parameters of the access token shall be determined by the CAPIF core function based upon the client_id of the API Invoker provided in the Access Token Request message.

The scope parameter ‘List of Services per AEF’ shall contain a full or partial list of services which the API Invoker is permitted to access at each AEF.

*************** End of 3rd Change ****************

*************** Start of 4th Change ****************

C.3.2	Access token request

To obtain an access token, the API Invoker makes a request to the CAPIF Core Function by sending an Access Token Request message with the following parameters using the "application/x-www-form-urlencoded" format, with a character encoding of UTF-8 in the HTTP request entity-body. The access token request parameters are shown in table C.3.2-1.

Table C.3.2-1: Access token request message parameters

| Parameter | Values |
| --- | --- |
| grant_type | REQUIRED. The value shall be set to "client_credentials". |
| client_id | REQUIRED. The identifier of the API Invoker making the request. It shall match the value that was assigned to the API Invoker during the onboarding process. |
| client_cred | OPTIONAL. The client credential that was provided to the API Invoker during the onboarding process. |
| scope | OPTIONAL. A string containing a space-delimited list, comprising of the following as scopes associated with this token: For general authorization: - List of Services per AEF (e.g. “AEF1:Service1,Service2,Service3,...,ServiceX; AEF2:Service1,Service2,Service3,...,ServiceZ”) For RNAA: - List of GPSIes of Services per AEF (e.g. “AEF1:Service1:GPSI1,...,ServiceX:GPSIx; AEF2:Service1:GPSI1, ,...,ServiceZ:GPSIz”) |

*************** End of 4th Change ****************