3GPP TSG-SA3 Meeting #112 	S3-233784

Goteborg, Sweden, 14 - 18 August 2023

Title:	LS on SEPP requirements, privacy and cyber security regulations

Response to:	GSMA overall LS

Release:	Release 18

Work Item:	TEI19

Source:	3GPP SA WG3

To:	GSMA NG, 3GPP SA1

Cc:	3GPP SA2

Contact person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	DocNumber(s) [Description e.g.. Draft TS 29.414 v0.1.0]. 
!! WARNING !! Do not insert the file directly as an object in this Word document.

# 1	Overall description

SA3 thanks GSMA for their LSs with additional requirements on 5G roaming to which 3GPP provided per SA plenary as overall response in SP-230763 that any new requirement for additional services is to be handled in a study in SA1 first as per 3GPP process.

The present LS is providing input to 5GMRR and SA1 for gaining a common understanding of SEPP in the scenarios given by 5GMRR:

- for GSMA as acknowledgement and request to respond to some technical questions, and

- for SA1 as input to a potential study and to check the use cases already captured in TR.33.875

This SA3 input is in addition to the LS input provided to 3GPP working groups in LS S3-233308 and the TR 33.875 documented key issues #10 and #12. For these key issues further handling in a separate study was concluded by SA3. It is recommended that SA1 checks, if those can be handled in parallel to studying new services as per GSMA request.

5G e2e service architecture

SA3 reminded GSMA in LS S3-231389 that any new requirement to be defined shall not break with the 5G e2e service architecture and the security defined for this architecture. A hop-by-hop roaming solution without the security defined for 5G would just reassemble the 2G/3G/4G roaming approach with intermediaries being able to modify control messages without operator knowledge. I.e., an intermediary can modify service requests of the NF Service Consumer sent from one operator network to the NF Service Producer in another operator network without technical proof option by the operator  of what an intermediary modified.

If readability and modification of messages is desired by intermediaries in 5G, an application layer security approach for roaming has to be applied. Breaking this principle allows an attacker at an intermediary hop to endanger the security and, at the same time, does not give operators and/or jurisdiction an easy technical possibility to track down who was responsibility for a message and/or its modification.

Privacy and cyber security regulations

The 5G security architecture design addresses regulations evolving world-wide on privacy and cyber security requirements, i.e., demanding e2e confidentiality and integrity protection between operators and allowing intermediaries to act (read and modify messages) only in an e2e operator-controlled way. By this, attributability in the sense of being able to know, which entity has sent or modified what, is provided by technical means and can therefore be used in legal disputes, if required by law. This needs to be kept in mind when SA1 is discussing new requirements for 5G deployments requiring, e.g., the hosting of a SEPP.

Definition of a 3GPP conform 5GC SEPP and N32-c/N32-f usage

Any proxy that is on the path between 2 PLMNs cannot be a SEPP by default. TS 33.501 cited:

"the SEPP – cf TS 23.501, clause 6.2.17 shall be present at the edge of the source and destination networks dedicated to handling e2e Core Network Interconnection Security. The confidentiality and/or integrity for the message elements is provided between two SEPPs of the source and destination PLMN".

N32-c is hereby used when the two SEPPs of the source and destination PLMN bilateral negotiate the N32-f protocol for sending and receiving control messages. N32-f can be either TLS if bilateral communication continues (without reading and modification options for intermediaries), or PRINS if intermediaries are allowed to read and modify selectively IEs from the control messages communicated between the two SEPPs to provide further services.

SA3 acknowledges the desire of RVAS, IPX and roaming hub providers to provide additional services. While RVAS and IPX may be able to offer this in 5G as a service within the operator security domain and by this maintaining the e2e service architecture requirement, considerations for necessary enhancements for roaming hubs are further discussed below.

Considerations on outsourcing and hosting SEPP deployment scenarios

SA3 acknowledges the desire of offering such service to operators that do not want to operate a complete 5GC system alone. The following summarizes the understanding of the terminology and provides links to existing specifications within 3GPP.

The term "hosting" has been introduced by GSMA for describing hosting a SEPP (on behalf of the PLMN operator) by a service provider (IPX or roaming hub), i.e., the NFs of the network operator connecting to this hosted SEPP. This implies that this hosted SEPP must belong to the security domain of the PLMN network operator with the link between NFs and hosted SEPP being protected by additional means. If a service provider is providing such hosted SEPP service to many PLMNs, separation of the PLMN's security domains needs to be guaranteed.

The term "outsourcing" has been introduced by GSMA for describing a SEPP in an operator group roaming hub shared between 'group operator member PLMNs' in an operator group hubbing scenario. Hence, it is a specific scenario for an operator group, where the security behind this hub is out of scope in 3GPP. In this case (SEPP being outsourced to an Operator Group Roaming Hub as described in 33.501, clause 13.1.2), the e2e core network interconnection security is provided between the Operator Group Roaming Hub SEPP and the destination network operator SEPP. A group network operator member PLMN's border element would not act as a SEPP if using the Operator Group Roaming Hub SEPP. But a proxy is necessary to transfer the NF service request of a group operator member PLMN to the Operator Group Roaming Hub SEPP securely. In addition, such group operator member PLMN can still have a SEPP that is interconnecting with a distinct set of other networks (inside the operator group and outside) that are not served by the Operator Group Roaming Hub SEPP.

Necessary enhancements for roaming hubs

As per 3GPP specification, application layer security needs to be applied if hops need to do more than IP level routing. To allow a public roaming hub to work smoothly, the operator could choose a security profile that allows the roaming hub to handle all its administrative tasks, e.g. all IEs necessary for the public roaming hub to fulfil its task are sent in clear from the initiating operator network SEPP to the receiving operator network SEPP.

If a roaming hub needs to create own control messages, architectural changes would be needed. Waiting for SA1 to finalize a study may be not in time for 5G SA deployments, therefore enhancements of  PRINS may be helpful. Requested for comments by GSMA already in LS S3-231389, the proposal S3-231419 of Enhanced PRINS to allow a roaming hub to also send its own control messages was provided to 5GMRR. An evaluation by 5GMRR for this enhancement would still be helpful. For SA1 to acknowledge: this update to PRINS would be a method that is in line with the 5GS e2e approach that underpins the current 5G security and trust model.

Alignment of existing 5GC SEPP requirements with the new GSMA requirements

SA3 would like to emphasise that any update on 5GC architecture needs to be in alignment with the existing 5GC SEPP requirements of 33.501, clause 4.2 and 5.9.3, and the solutions described in clause 13.1.1, 13.1.2,13.2, and 13.5. And if not, SA1 needs to provide guidance how contradicting requirements should be handled.

If hop-by-hop solutions are still the preferred way forward by GSMA stakeholders in the roaming ecosystem, SA3 requests to consider the continuation of roaming with the existing 2G/3G/4G roaming solutions, to enable a 5G System to be compatible with privacy and cyber security regulations if required.

# 2	Actions

To: GSMA 5GMRR, 3GPP SA WG3

ACTION:

SA3 asks 5GMRR to provide feedback on Enhanced PRINS, to acknowledge the role of a 5GC SEPP, and to confirm the understanding of hosting and outsourcing. Please inform on any updates happening in GSMA.

SA3 asks SA1 to take above into consideration when setting up and studying new requirements for 5GS. Please check if TR 33.875 key issues related to roaming can be studied from security perspective in parallel.

# 3	Dates of next TSG SA WG 3 meetings

SA3#113	6 - 10 November 2023	Chicago, US

SA3#114	22 - 26 January 2024	EU (TBD)