3GPP TSG-SA3 Meeting #112	S3-233884

# Gothenburg, Sweden, 14 - 18 August 2023

# Source:	Qualcomm Incorporated

# Title:	Updates on discovery procedures

# Document for:	Approval

Agenda Item:	4.26

# 1	Decision/action requested

This contribution proposes to update the discovery procedures in the draft CR.

# 2	References

[1]	TS 23.586

[2]	S3-233311

# 3	Rationale

This contribution proposes to update the clause 6.2 (Security for Ranging/SL positioning discovery) in the draft CR to make the terms aligned with TS 23.586 [1].

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the draft TS [2].

***** START OF 1st CHANGES *****

## 6.2	Security for Ranging/SL positioning UE discovery

### 6.2.1	General

For ProSe capable UEs, the discovery procedures of both Model A and Model B defined in clause 6.3.2 of TS 23.304 [4] are used for Ranging/SL Positioning UE discovery.

For V2X capable UEs, the procedures for V2X communication defined in clause 6.3.3 of TS 23.287 [5] are used for Ranging/SL Positioning UE discovery.

### 6.2.2	Security requirements

The 5G system shall support integrity protection, confidentiality protection and anti-replay protection of discovery messages.

The SLPKMF or 5G PKMF shall be able to provision discovery security materials to ProSe capable UEs. The discovery security materials are used to protect the integrity of discovery messages and privacy sensitive information (e.g. UE identity) in the messages.

The ciphering algorithm for discovery message confidentiality shall be configured by the network during discovery key request procedure.

Editor’s Note: Whether a new network function (SLPKMF) or an existing network function (5G PKMF) is used is FFS.

### 6.2.3	Security procedures for ProSe capable UEs

The security mechanisms for both models of restricted 5G ProSe Direct Discovery defined in clause 6.1.3.2 of TS 33.503 [6] are reused for ProSe capable UEs to provide protection for Ranging/SL positioning UE discovery.

The main difference is that SLPKMF or 5G PKMF rather than 5G DDNMF is used to provision discovery security materials for Ranging/SL positioning UE discovery.

***** END OF 1st CHANGES *****