3GPP TSG-SA3 Meeting #112 	S3-233925

# Goteborg, Sweden, 14-18 August 2023											   (revision of S3-yyxxxx)

# Source:	Ericsson

# Title:	Resolving Editor's Note on Authorization of selection of participant NWDAF instances in the Federated Learning group

# Document for:	Approval

Agenda Item:	4.23

# 1	Decision/action requested

Approve the proposed changes to the living document [1] below.

# 2	References

[1]	S3-233266 "living CR for eNA"

[2]	3GPP TR 33.738 "Study on security aspects of enablers for Network Automation for 5G - phase 3"

[3]	3GPP TS 23.288 " Architecture enhancements for 5G System (5GS) to support network data analytics services"

[4]             S3-233923, Ericsson, "Resolving Editor's Note on Interoperability indicator of model storage consumer"

# 3	Rationale

The procedure explaining Authorization of selection of participant NWDAF instances in the Federated Learning group contains the following Editor's Note:

Editor’s note: The use of Service area, interoperability indicator and Availability time requirement for authorization is FFS

This change request to the living document [1] proposes to remove the Editor's Note, with the following reasoning:

According to Clause 5.2 in [3], Service area and Availability time requirement (i.e. Time interval supporting FL) are required for NWDAF Discovery and Selection. But for authorization of selection of participant NWDAF instances in the Federated Learning group, Service area and Availability time requirement are not essential because these parameters are about availability instead of authorization.

About interoperability indicator, it is explained in [4].

# 4	Detailed proposal

*** START CHANGES ***

# X.9	Authorization of selection of participant NWDAF instances in the Federated Learning group

The authorization for selecting participant NWDAF instances in the Federated Learning (FL) group uses token-based authorization as specified in clause 13.4.1, with the following additions.

Figure X.9-1 depicts the authorization mechanism for NWDAF containing MTLF acting as FL Server to initiate the Federated Learning process on the NWDAF containing MTLF(s) acting as FL Client(s). The authorization is based upon the FL capability type (FL server or FL client) provided by the NWDAF containing MTLF acting as FL server during registration, and the Analytics ID and Interoperability Indicator per Analytics ID provided by the NWDAF containing MTLF acting as FL client during registration.

Figure X.9-1: FL Authorization for selecting participant NWDAF instances

Editor's Note: The Figure needs to be aligned with the procedure.

Step 1a. The NWDAF containing MTLF acting as FL client registers to the NRF with its FL related information, including supported FL capability (FL client), Analytics ID(s) and Interoperability Indicator per Analytics ID as described in clause 5.2 of TS 23.288.

Step 1b. The NWDAF containing MTLF acting as FL server registers to the NRF with its FL capability (FL Server).

Step 2. The NWDAF containing MTLF acting as FL server (NF Service Consumer) sends a discovery request to NRF and receives the available NWDAFs containing MTLF acting as FL client(s) (NF Service Producer) as a response, as specified in clause 6.2C.2.1 of TS 23.288 [105].

Step 3. The NWDAF containing MTLF acting as FL server (NF Service Consumer) sends an access token request to the NRF  as specified in clause 13.4.1. The access token request may contain the Analytics ID for the requested Federated Learning process.

Step 4. The NRF authorizes the NWDAF containing MTLF acting as FL server (NF Consumer) based upon the information received in Step 1a, and after verifying that the Server NWDAF’s Vendor ID is included in the Interoperability Indicator for the requested Analytics ID provided in Step 1b. If the authorization succeeds, NRF generates the access token(s) as specified in clause 13.4.1. The access token claims may include the Analytics ID for the request Federated Learning process.

Step 5a, 5b. The NRF sends the access token to the NWDAF containing MTLF acting as FL Server, or rejects the request in case of failed authorization, as described in clause 13.4.1.

Step 6. The NWDAF containing MTLF acting as FL server sends the service request to the NWDAF(s) containing MTLF acting as FL client with the access token received in Step 5a. along with the Analytics ID information for which the FL process is to be performed, as described in TS 23.288 [105].

Step 7, 8. The NWDAF containing MTLF acting as FL server (NF Service Producer) verifies the received access token as specified in clause 13.4.1. In case of successful access token verification, the NWDAF containing MTLF acting as FL server sends a success response to the NWDAF containing MTLF acting as FL server, as described in TS 23.288 [105].

Step 9. After a sucessful response from the NWDAF(s) containing MTLF acting as FL server, the NWDAF containing MTLF acting as FL server initiates the Federated Learning process as described in TS 23.288 [105].

Authorization of the NWDAF containing MTLF acting as FL client is implicit, since it can join a Federated Learning group only when selected by the NWDAF containing MTLF acting as FL server.

*** END CHANGES ***